-----------------------------------------------------------------

|

A discussion of |

|

C O M P A T I B I L I T Y |

|

Expanding your system and |

exploring new worlds |

|

-----------------------------------------------------------------

A  lot  of ATARI users are concerned about the  compatibility  of

their software with our high resolution true color boards.

Compatibility  is  a key issue when attempting  to  improve   the

system performance through hardware upgrades.  This is especially

true  for graphic cards.  The ability to expand a system  greatly

depends  on  the  way  how  the  operating  system  handles   I/O

operations (like producing an output on the  screen),  additional

hardware and the accompaning software drivers.

Fortunately,  the  ATARI computers feature GEM.  GEM consists  of

several software layers, each providing an increasing freedom and

independence from the underlying hardware components.

One of these GEM layers is called VDI (Virtual Device Interface).

By exchanging the existing ATARI specific VDI portion of the  GEM

environment,  the  ATARI  system can accommodate a wide  range of

other display devices.

Therefore we have not only tried to develop a unique and powerful

graphics expansion board but also a VDI driver that emulates  the

original,  which  maintains a high degree of compatibility  while

extending  the  capabilities according to the  provided  hardware

features.

Considering these facts,  all programs can be divided into  three

groups:

a) programs that use the VDI as the only interface to produce ANY

output on the screen.

b) programs that seem to require one of the original Atari  modes

to run properly.

c)  programs that use the VDI AND CUSTOM ROUTINES to  update  the

display.

Case  'a'  programs  show little or no difference  when  used  in

conjunction  with  the CyReL M16-1280 boards.  Almost  all  major

software  titles can be found in this group.  These  applications

allow the user to take advantage of e.g.  higher resolutions  (in

some cases even higher refresh rates),  more colors and sometimes

even faster output speeds.  Fortunately most of them are designed

according  to the GEM and ATARI guidelines and therefore  can  be

used instantly without any additional system modifications.

Case  'b' programs are a little bit more difficult to deal  with.

Most of these programs use the information of the Getrez (XBios4)

operating  system  call to find out more about the state  of  the

display.   Instead  of  examining  the  (correct  and  available)

informations provided by the VDI some programmers seem to  prefer

the XBios call. This can lead to some surprising results:

THE  CALL  CAN ONLY RETURN A CODE TO DESCRIBE THE  GRAPHICS  MODE

WHICH  IS  CURRENTLY  BEING  USED BY  THE  BUILT-IN  ATARI  VIDEO

CIRCUITRY!

Original  intention:  a program wants to find out if the  current

screen  resolution supports all the requirements (e.g.  at  least

640x400)

Effect:   Some   programs  refuse  to  run  even  when  using   a

sufficiently  high resolution because it might be that the  ATARI

VIDEO CIRCUITRY is operating in a lower mode,  and thus the XBios

call returns an unacceptable value.  Note that we support a  'two

monitor' configuration, consisting of our graphics card connected

to  one  monitor and the simultaneous use of  the  screen  output

generated by the Atari connected to a second monitor.  WE DO  NOT

IMPOSE  ANY RESTRICTIONS WHATSOEVER regarding the mode  in  which

the  Atari must be placed.  Our cards even allow  'multi-monitor'

setups with up to four CyReL cards per single system + the  Atari

output.

Patch:  We provide custom XBIOS driver that can be installed plus

an  accessory that allows the user to define the return value  of

the XBios call to eliminate about 90% of the described problems.

Nonetheless  we would like to urge EVERY developer to  adopt  the

VDI method for any inquiries regarding screen size and/or  screen

attributes!

Further  we  have  to add all programs which  take  advantage  of

certain  hardware  register layouts and ATARI  specific  hardware

features  to group 'b'.  The use of such features prevents  these

programs from being used with a graphics card. A    number     of

monochrome-only drawing programs and almost all of the games  and

entertainment software titles fall into that category.

This  is  not limited to CyReL cards and applies  to  all  system

expansions  as  long as they are not  specifically  supported  by

those programs.

Group 'c' programs can prove to be the hardest to adapted by  the

user to run on any graphics card, since some of the screen output

may  end  up  at the wrong place and most  likely  in  the  wrong

format. There is little the user (or we) could do to correct this

problem,  which  means that in almost all cases the developer  or

author of the program must re-write at least a part of the  code.

The only way to prevent such problems is to fully support the VDI

interface   from  the  very  beginning.   With   the   continuous

development and introduction of new ATARI systems,  it represents

the  only way to maintain compatibility with new  ATARI  features

and ultimately all third party products.

Further  it  is  the only way to  guarantee  the  portability  of

programs  in case the user intends to upgrade to a bigger system.

Obviously  there  will  be future  developments  that  cannot  be

forseen  and  thus will remain unsupported  by  today's  software

titles.  But we can at least do our share to improve  flexibility

and user-friendlyness for years to come.

This  adaptation of programs is a process that has already  begun

and benefits all, the user, the developer and the manufacturer of

e.g.  the graphic boards. The developer can take advantage of new

improved screen output,  a higher number of on-screen colors, and

not to forget new and evolving market niches.  The user  benefits

through improvements in appearance,  comfort, features and better

ergonomics   while  the  manufacturer  can  take   advantage   of

increasing market opportunities.

We at Cybercube welcome the introduction of the Falcon. We see it

as  an  interesting and important addition to the  Atari  product

line.  It will help create a market that is increasingly oriented

towards graphic and video applications.  Furthermore it will help

educate  the user base in regards to the requirements  of  modern

desktop   video   technology.   This   aspect   should   not   be

underestimated.

It is our goal to provide a quality product for the  professional

user or the serious home user. With on-screen resolutions of e.g.

2048x1024 in 256 colors, refresh rates that can go beyond 100 Hz,

real  (no  less than 16 Million colors) True  Color  capabilities

with  resolutions  of e.g.  640x480,  800x600,  960x512  or  even

1024x512 and virtual resolutions up to 4096x4096 (non-interlaced)

we  do not anticipate any problems regarding the evolving  Falcon

market.

Through the additional expansion capabilities of the cards (which

is  for instance  being  utilized by  the CyReL VidiMix8  desktop

video module) we think that we have  a  very  promising  platform

to  offer.  A  platform  that  easily  integrates  into the  ever

expanding Atari environment.

But we are still convinced that there is a need of a co-operation

between users,  software-developers and hardware manufacturers in

order to make this concept work.  We hope that our  contributions

might  help  to  establish  the  Atari  computers  as  a   viable

alternative  to the popular systems in the DTP,  CAD and  desktop

video markets.

In  this spirit,  I would like to thank you for your interest  in

our products.

Best regards,

CYBERCUBE RESEARCH LTD.

M16-Product Coordinator

Please  feel  free to write,  e-mail or fax us  any  comments  or

suggestions.  We are interested in your feedback and will do  our

best to make needed improvements on a timely basis.