---------------------------------------------------------------------------

CyReL SUNRISE M16-1280 THE M16 PALETTE DATA FILES

Version: 1.0a                           Date:1/92

Version: 1.1b                           Date:7/92

---------------------------------------------------------------------------

CONTENTS

A.   Introduction

B.   Format

i.  General Description

ii.  Summary

C.   How to Decode the M16-PAL1 Palette Data Files/Blocks

i.  M16-PAL1 Format

D.   Notes

---------------------------------------------------------------------------

[A] INTRODUCTION

---------------------------------------------------------------------------

The CyReL M16-1280 Palette Data Files

The CyReL M16-1280 graphic cards support a wide variety of modes

and resolutions. It was therefore necessary to create a standard

yet flexible way to store the associated informations.

This file describes the format of the M16 Palette Data Files and

shows how to decode them. It's an easy and straightforward process.

Simply follow the instructions given below.

---------------------------------------------------------------------------

[B] FORMAT

i. General Description

---------------------------------------------------------------------------

The Palette Data Files consist of data blocks. Every block has a

QWORD header. The header is being followed by a LWORD link vector

pointing to the next block. The link vector of the last block

contains 0x00000000 to indicate the end of the linked list of

data blocks.

Every header starts with the ASCII character string "M16-" and

is being followed by the category ID, consisting of another four

characters and specifies the nature and structure of the data in

the block itself.

The decoding of the data block varies from category to category

and all formats of additional blocks will be published at time of

their release.

---------------------------------------------------------------------------

ii. Summary:

---------------------------------------------------------------------------

File Offset: + 0x0000   "M16-"          Standard header.

+ 0x0004   "xxxx"          4 Characters specifying the

category.

+ 0x0008   0xnnnnnnnn      Pointer to next header or

0x00000000.

Contains the nr of following bytes

(in the file) that must be skipped

in order to get to the start of

the next header. A value of zero

indicates that this is the last

block in the file.

+ 0x000C   dd dd dd ...    Start of data block

---------------------------------------------------------------------------

[C] How to Decode the M16-PAL1 Palette Data Files/Blocks

i. M16-PAL1 FORMAT:

---------------------------------------------------------------------------

LABEL/CONTENTS  SIZE    DESCRIPTION

"M16-PAL1"      <Q>     Complete QWORD ASCII header.

0x????????      <L>     Link vector to next block or zero.

start_reg       <W>     First color register to be loaded

with the following color data.

nr_of_regs      <W>     Number of following color data fields.

cdata0_red      <W>     Color field zero, RED component

Range: 0-1000

cdata0_green    <W>     Color field zero, GREEN component

Range: 0-1000

cdata0_blue     <W>     Color field zero, BLUE component

Range: 0-1000

cdata1_red      <W>     all additional fields (if any)

.                       Refer to the nr_of_regs entry

.                       for the total nr of color data fields.

.

---------------------------------------------------------------------------

[D] Notes:

---------------------------------------------------------------------------

This file will be appended from time to time. Please report any

new block releases to our M16-Product Coordinator.

---------------------------------------------------------------------------

<EOF>