PMMUPATCH 2.01 -- New version (Updated November 13, 1991)

First, this utility is freeware. You are granted the right to use either

the program or the source code. If you want to register, try to send a

postcard to the address at the end of this file.

The program uses the PMMU to map the 32 bit address of the 68030 into

the smaller and standard address range of the 68000, i.e. 24 bits. This

should help with programs that use the upper byte of pointers to store

extra information (e.g. all GFA written programs). A new improvement

remaps also the area $fc0000-$feffff into $e00000-$e2ffff ; this could

be useful with programs that attempt to read directly the ROM (often to

get TOS version).

However, this is *NOT* a miraculous masterpiece, so *DO NOT* expect any

program to work perfectly well. You will surely get problems if you have

TT Ram, for TT Ram is installed above the standard 16 Mb. This could lead

to partial overlap between ST and TT Ram ; in such a case, only the ST Ram

will be accessed. Thus, don't use this patch with programs which use TT Ram

(Such as TC 2.03 or Pure C) : you have a very high probability to crash

the system ; anyway, such programs are written correctly and do not need

the patch to execute correctly.

I have not made many extensive tests on programs known to crash ; if you

have a file that bombed before, but works fine with the patch, please

could you send me the name of the program ? That would help me supply with

a newer release of the patch a list of corrected programs.

The next version will provide a hot-key to dynamically switch between the

patched MMU configuration and the standard one, to improve compatibility

with TT Ram using utilities. This should be released soon.

Vincent Habchi

ENST - Dpartement informatique

56, rue Barrault

75013 PARIS

France

EMAIL : Elendir@inf.enst.fr