---------------------------------------------------------------------------

CyReL SUNRISE M16-1280 TC_SHIFT.PRG Documentation

Version: 1.0a                          Date: A/93

---------------------------------------------------------------------------

CONTENTS

A.   Questions & Answers

B.   NOTES

---------------------------------------------------------------------------

[A] QUESTIONS & ANSWERS

---------------------------------------------------------------------------

Q: What does TC_SHIFT.PRG do ?

---------------------------

A: TC_SHIFT instructs the CyReL SUNRISE M16-1280 hardware and software

drivers to emulate the alpha-RGB True Color mode (as used by the Matrix

graphics cards). The CyReL M16-1280 cards normally work in the RGB-alpha

True Color layout.

Q: How do I Use TC_SHIFT.PRG ?

---------------------------

A: Simply run the TC_SHIFT.PRG before you run any of the applications that

need the mode adjustment. One such program is e.g. the Studio Photo program

from EuroSoft/COMPO.

Q: Which Programs do Require the TC_SHIFT.PRG?

-------------------------------------------

A: As stated before, use it in combination with COMPO's Studio Photo.

You can also use it with Calamus or Outline-Art.

Q: How do I Install TC_SHIFT.PRG ?

-------------------------------

A: This is the first release of the TC_SHIFT.PRG. If you want to make it's

use a little simpler, why not install it so that it can be run whenever you

press a certain function key! Future versions will allow you to customize

the installation for certain programs.

Q: When do I have to Use TC_SHIFT.PRG ?

------------------------------------

A: Use it every time you want to run any of the above listed

programs in True Color mode. The TC_SHIFT.PRG program will

check the present configuration and only execute a short

subroutine if all of the following conditions are met:

* there must be at least one installed CyReL M16-1280 card.

* the revision level of the card must be Rev 2F or higher.

* the revision level of the driver software must be V1.7b

or higher.

* the card must be operating in 24bit/pixel or 32bit/pixel

mode (true color).

* the CyReL True Color VDI must be running AND using that

particular card.

Note:

Refer to the additional documentation on how to change to the true

color mode and back.

Q: Are there any known side-effects ?

----------------------------------

A: So far, none have been found or reported.

Q: I have launched my programs either from the desktop (icon

---------------------------------------------------------

or function key) or another program. What do I have to change ?

---------------------------------------------------------------

A: Install the TC_SHIFT.PRG program as either an AUTO-LAUNCH (will

be started every time you boot GEM) or as an F-key program. In

case you installed it as a function-key launched program, simply

press the corresponding F-key before running your main application.

Q: How much memory does TC_SHIFT.PRG actually use ?

------------------------------------------------

A: TC_SHIFT.PRG is a very small program. And it does not require more

than 3KB for proper operation.

Q: Is TC_SHIFT.PRG a TSR (terminate and stay ready) program ?

----------------------------------------------------------

A: Yes, once it is installed, you do not need to call it again (unless

you have reset your computer in the meantime).

---------------------------------------------------------------------------

[B] NOTES

---------------------------------------------------------------------------

Copyright (c) 1993 Cybercube Research Ltd, Canada.

All rights reserved.

---------------------------------------------------------------------------

<EOF>