G8BPQ Switch Driver Programs.

This file documents the driver programs provided with the switch. They fall

into two categories, programs to interface to external hardware not supported

directly by the switch, and programs to interface application software to

the switch.

Hardware Drivers.

These programs use the EXTERNAL port interface (see EXTERNAL.DOC). This

interface was provided primarily to allow users to write their own hardware

drivers, but I have started to use it to support hardware which is likely to

be only of minority interest, so dont justify occupying memory unless required.

So far there is only one driver in this category.

ODIDRV

This program allows switches to be linked via Ethernet. The software runs

over the Novell ODI interface, so should work with any Ethernet card for

which an ODI driver is available.

Application Software Drivers.

From Version 4.00, the application interface to the switch has been changed.

The new interface is called BPQ Host Mode (see BPQHOST.DOC for full details).

Some software is available for the new interface (eg F6FBB BBS, TERM4), but,

in order to support older software, and to provide new facilities, a number

of drivers have been written. These are TSR's, which are loaded after BPQCODE.

The drivers supplied with 4.06 are detailed below:

BPQHTNC2

This provides the COMBIOS/TNC2 emulation, as used by W0RLI and G1NNA

BBS's (amongst others), and various Terminal Programs (YAPPB, PAC2 etc).

BPQHKISS

This provides the COMBIOS/KISS emulation. This was used for the NET

TCP/IP interface, and for programs like STATS and TLMDC (the Satellite

telemetry decode program).

DEDHOST

This provides an emulation of the WA8DED Host Mode driver used with DRSI

cards (TNCTSR). It is used for PacketCluster and AA4RE BBS.

NODEDRV4

This is used to interface the NOS TCP/IP package to the switch. It conforms

to the PacketDriver interface spec.

Full details of the above drivers are given below:

ODIDRV - Ethernet Interface

This driver allows two or more switches to be linked via Ethernet. The driver

uses the ODI system to interface to the ethernet card. A working knowledge

of ODI is assumed in this document.

The driver has one parameter, the software interrupt to be used by the

switch to communicate with the driver. This must match the INTLEVEL param

in BPQCFG.TXT. The number is specified in decimal in both places:

ODIDRV 125

By default, the system uses a multicast address of 01:42:50:51:0:0, and

I recommend the use of ETHERNET_II frame type, and PID=08FF. These can

all be overridden by parameters in NET.CFG

NET.CFG Params:

LINK SUPPORT

MAX STACKS 4

MAX BOARDS 4

LINK DRIVER NE1000			; or other MLID to suit your card

INT 2				;

PORT 300				; to suit your card

FRAME ETHERNET_802.2		; may not need all these

FRAME ETHERNET_II

FRAME ETHERNET_802.3

FRAME ETHERNET_SNAP

PROTOCOL IPX 0 ETHERNET_802.3	; if you want to use Netware as well

PROTOCOL BPQ 8FF ETHERNET_II	; required for BPQ - can change PID

BPQPARAMS				; optional - only needed if you want

; to override the default target addr

ETH_ADDR  00:80:C7:4A:97:03	; Target address. See below

By default, the system uses multicasts. This simplifies configuration, and

allows several nodes to communicate. If you only have two nodes you can

improve efficiency marginally by specifying the hardware address of the

other ethernet card. You can also specify the broadcast address

(ff:ff:ff:ff:ff:ff). This may be necessary if your MLID doesnt support

multicast.

I havnt found any problem using a PID of 8FF, but you can change this if

necessary.

TNC2 Emulation routine - BPQHTNC2

This program provides the application interface for TNC2 applications

on switch version 4.05 and above. BPQHTNC2 should be loaded after the switch.

If only a single program uses the TNC2 interface, (eg NNA), it may be loaded

in the programs's Desqview window, but if it is used by more than

one program (eg RLI, or utilites such as PAC2, BPQTERM, MH), it should be

laoded before Desqview. If in doubt, load it before DV.

The program may be called in two ways. If called without any paramters, it

will use the TNCPORT information from BPQCFG.BIN, and use BPQHOST ports 33

upwards. This should be adequate for the great majority of applications.

Alternatively it is possible to supply 3 parameters, the number of TNC2 ports

required, the first COMBIOS port required, and the first BPQHost port to use.

This will only be necessary if you want to use more than 16 TNCPORTS, or if

you are already using BPQHOST ports above 33.

For example,

To use the ports defined in BPQCFG.TXT (ie to be the same as earlier versions)

BPQHTNC2

To create COM1-COM18, enter:

BPQHTNC2 18 1 1

For COM2-COM5

BPQHTNC2 4 2 1

A single copy of BPQHTNC2 can only support a contiguous set of COM ports. If

you want set up say COM1-COM4, then COM8-COM9, you can run more than one

copy:

BPQHTNC2 4 1 1

BPQHTNC2 2 8 5		(Four BPQHOST ports have been used by first)

There is some overhead in running two copies, so in general it would be

better to rearrange the applications to use a contiguous set of COM ports.

The last parameter will normally be 1. Only if you are using BPQ Host ports

for other software (eg FBB, TERM4, DEDHOST), or are running more than one copy

of BPQHTNC2, will it need changing.

Please note that BPQHTNC2 reads BPQCFG.BIN to find what interrupt to use

to talk to the switch, and to get the default value for PACLEN. BPQCFG.BIN

must therefore be in the current directory when BPQHTNC2 is loaded.

The following commands are supported by BPQHTNC2:

CONOK ON/OFF	- Indicates the port is available for use. By default, it

will be for the BBS application. See APPL comand.

APPL		- Bit significant field, to indicate which application the

port is supporting. The first three are reserved for BBS,

connects to the SYSOP (HOST support), and remote SYSOP

facilities. The other 5 are available for you to define your

own application servers (eg DATABASE, UNIX gateway).

Default is 01, the BBS.

To allow users to connect to you (using the C command

from the switch without a param), set APPL=02, and CONOK ON

BBSMON ON/OFF	- Reduces the monitor output to calls only (enough to maintain

the JA list). Place in the MBL config file startup section.

MTX ON/OFF	- Monitors transmitted frames - mainly used for debugging

MONITOR ON/OFF	- As in normal TNC

MALL ON/OFF

MCOM ON/OFF

MCON ON/OFF       Note that to monitor whilst connected to the Node, you

need MCON ON

MMASK 		- Selects which (radio) ports will be monitored. A bit

significant field - setting bit 0 allows monitoring of

the first port, bit 1 the second, and so on. Default is

hex FF, monitoring all radio ports. Setting it to zero

will suppress all monitoring.

The monitor code will decode and display the network headers from messages

between nodes. This enables you to see what routes are being used between

nodes, as well as removing a lot of garbage from the monitor display!

NOMODE ON/OFF	The next 2 are accepted, but are not fully implemented. In

NEWMODE ON/OFF  general BBS connects assume NOMODE on, others NOMODE off.

CONNECT

To connect to the node, use the command NODE (or K). This is equivalent to

connecting to a normal node. There is a (roughly) equivalent command

C SWITCH, which returns the string *** CONNECTED to SWITCH, and is used

in MBL forward files, as the MBL software is expecting the first connect

to be at Level 2. A Connect command without a parameter returns the

current state, again mainly for use by MBL software. A connect with any other

parameter connects to the Node, then passes the command to the Node's command

processor, thus allowing you to enter a normal Connect from either TNC or NODE

command level.

CONV

TRANS

UNPROTO CALL [VIA CALL ...]

BTEXT

The CONV and TRANS command enter the appropriate state, and if not connected

to the Node, allow you to send UI frames. This is used for the MBL 'Mail for

xxxx' broadcasts, and can be used for CQ's , etc. Beaconing is not supported,

but the BTEXT text is saved, and may be send instead of the predefined ID

message at intervals. This is primarily for RLI 'MAIL' beacons.

Note that K is an abbreviation for NODE, not CONV, as in a normal TNC, as

normally you want to switch easily between TNC command and Node command modes.

SENDPAC

CPACTIME

CR		- As normal TNC.

FLOW ON/OFF	- Similar to normal TNC, but a bit simpler. in particular

echoing is suppressed, so if you need to use FLOW ON, I

suggest you set ECHO OFF

Background information on INT14 Drivers.

Both BPQHTNC2 and BPQHKISS emulate the COMBIOS/MBBIOS interface used by

WA7MBL, AA4RE and others.

The following contains information extracted from AA4RE's MBBIOS

Documentation. It has been modified to conform to my COMBIOS

emulation.

MBBIOS supports the standard INT 14 calls plus the extended ones as

defined by Jeff Jacobsen, WA7MBL for his BBS program.  These are

explained below.  AH=0 to AH=3 are identical to the IBM BIOS definition.

DX must contain the port number, which start at zero, so for COM1 set

DX=0, COM2 set DX=1, etc.

AH = 0     Initialize

---------------------

This is provided for compatibility only. It has no effect except to

return the current (virtual) LSR in AH, bit zero of which is set if

any characters remain in the rx buffer, and virtual MSR in AL.

The MSR bits supported are DCD=Connected

CTS=Not busy

An attempt is also made to maintain delta bits

BIT 7 - Receive Line Signal Detect (DCD)

6 - Ring Indicator (Not supported)

5 - Data Set Ready (Always on)

4 - Clear To Send

3 - Delta  RLSD ( ie state has changed since last

access)

2 - Not Supported (Trailing Edge Ring Detect)

1 - Not Supported (Delta DSR)

0 - Delta CTS

AH = 1     Send Character

-------------------------

AL contains the character to send.  Return values are the same as above.

AH = 2     Receive Character

----------------------------

On return, AL contains the character received.  AH contains the

virtual LSR. Again, AH bit 0 indicates if there is additional characters

in the buffer.  This routine does NOT timeout while waiting for a character

but will hang indefinitely waiting for a character to appear.

AH = 3     Return Status

------------------------

Return values are the same as returned by the Initialize routine.

AH = 4     Inquiry

------------------

Returns AX and BX set to 0AA55H.  This call is used to see if the

particular port is defined in TNCPORTLIST. Note that COMBIOS/MBBIOS

set AX, but not BX

AH = 5     Drop RTS

-------------------

This drops (virtual) DTR and RTS.  Will stop higher level code sending

any more data into the receive buffer. No return value.

AH = 6     Raise RTS

--------------------

Raise DTR and RTS.  No return value.

AH = 7     Send Break

---------------------

Generates a "Break".  Returns the virtual TNC to Command mode from Converse

or Transparent mode. No return value.

AH = 8     Non-destructive Read

-------------------------------

Returns the next character in the buffer in AL without removing it from

the buffer.  AH value defined as in AH=2 so you can use the Data Ready

bit to check to see if you really did get a character.

AH = 9     Set/Get options

--------------------------

Not implemented.

AH = A     Write buffer

-----------------------

Sends CX characters starting at DI:ES.  Upon return, DI will be updated

and CX will contain the unsent character count.  Unsent characters could

result from either BUFFER FULL or hardware handshake is OFF.  AX will

be set as in AH=3 (Status) above.

AH = B     Read buffer

------------------

Receives up to CX characters starting at DI:ES.  Upon return, CX set to

actual count read and DI is updated.  If the buffer is empty, return is

immediate with CX = 0. The function will return a 'logical' line, up

to a CR in normal mode, or 17H in Host Mode. On return, AL contains the MSR

and AH has the virtual LSR. Again, AH bit 0 indicates if there are additional

characters in the buffer.

AH = 1F - G8BPQ NODE-SPECIFIC COMBIOS EXTENTIONS

------------------------------------------------

AL = 0 - Return Version and Node DS

Returns: AX=Node data segment

DH=Major Version

DL=Minor Version    - ie for version 3.50, DH=3, DL=50

AL = 1 - Return ES:SI pointing to connected call (in AX25 firm), or

SI=0 if not connected. If connected, returns session type

in AH, and port number (if L2 session) in AL. Session

type is a bit significant field defined as follows:

;

;	CIRCUITTYPE EQUATES

;

L2LINK	EQU	1

SESSION	EQU	10B

UPLINK	EQU	100B

DOWNLINK	EQU	1000B

HOST		EQU	10000B

AL = 2  Return (in AH) number of unacked packets on connection.

Some versions (pre-4.05f) incorrectly returned the value

in AL instead of AH. To maintain compatibility with software

written to allow for this, the value is now returned in AL

as well as AH.

AL = 3  (Versions 4.05f and later)

Set PACLEN to the value recommended by the node. This will

initially be derived from the type of connect (ie L2 or L4),

and the port used (if L2). Eventually it will be varied to

optimise throughput, based on link performance.

AL = 10H Disconnect user form virtual TNC, and return user to Node

The segment registers are saved, (unless retuning a value), but other

registers may be destroyed.

WA8DED Host Mode Interface for G8BPQ Switch.

A facility has been developed to allow applications which normaly operate

in DED Host Mode, using a DRSI card and the DRSI support routine TNCTSR,

to be used with my switch code. An external interface routine DEDHOST.COM

is used to emulate the DRSI support routine TNCTSR. DEDHOST must be run after

BPQCODE. If running with Desqview, DEDHOST should be run in the same window

as the application. Version 4.01 (or higher) of BPQCODE is required. Note that

my code emulates a single DSRI port - multiple comms links may be connected to

the switch, but as far as the application software is concerned, there is a

single (virtual) link to the switch, capable of supporting up to 32 sessions.

DEDHOST.COM requires 5 parameters, all are decimal numbers. A 6th is optional

- if present, it is a single character. Parameters are:

Number of Streams.

This defines the maximum number of simultaneous users. The maximum is 32.

Start Stream.

The first stream number to use. The switch supports 64 streams. You must

ensure that each application running with the switch uses a different range

of streams.

Application.

The number allocated to this application in the APPLICATIONS param of

BPQCFG.TXT. Will normally be 1 if running a BBS or similar system.

Buffer Space.

The space to be allocated for buffering messages. You can specify up to 60

(Kilobytes). The program will allocate a minimum of 8k even if you specify

less. The space needed will depend on your traffic, and the software you are

using. I suggest you allocate as much as you can spare. If the application

then runs short of memory, you will have to experiment to find the optimum

value for your installation.

Interrupt Number.

The Interrupt used by the application to communicate with DEDHOST. Will be

255, unless you need to use that for some other software.

Autotimer Flag.

Normally the driver is configured to request the switch to send a message

after 11 minutes without activity on each stream. This is so that

PacketCluster users may remain connected during perions of low activity. If

you dont want this facility, or are not using Packetcluster, set parameter to

'N'. If you omit the parameter, or enter any other value, it will be enabled.

A typical command line would be DEDHOST 32 1 1 60 255 Y

Please note that DEDHOST reads BPQCFG.BIN to find what interrupt to use

to talk to the switch. BPQCFG.BIN must therefore be in the current directory

when DEDHOST is loaded.

NOS/BPQ Link - NODEDRV4.COM

The 'Packet Driver' I wrote to allow NOS to communicate with the switch

has been modified to use the BPQ Host mode interface. One copy of the

packet driver is needed for each radio port to be used by NOS, plus one

for the loopback port if you want NOS to be able to talk to the switch.

I dont have a lot of experience of using NOS, but I suggest that if you

are using TCP/IP over NETROM that you use the lookback port, and let the

switch handle all the radio links. Only if you are using IP in datagram

or ax.25 virtual circuit mode will you need to give NOS direct access to

the radio ports.

Please note that you need at least version 4.03 of BPQCODE.EXE

The following is an extract from my system. You will need to tailor

it to your own requirements.

If using Desqview, NODEDRV4 must be loaded in the same window as NOS -

if you put it under desqview the system will probably crash. It takes

three parameters, the software interrupt number used by NOS to communicate

with it (in the range 60-7f hex), the BPQ Host port to use to talk to the

switch (1 - 63), and the number of the radio port to be accessed.

Please note that NODEDRV4 reads BPQCFG.BIN to find what interrupt to use

to talk to the switch. BPQCFG.BIN must therefore be in the current directory

when NODEDRV4 is loaded.

STARTNOS.BAT

nodedrv4 0x61 33 1 		I am using BPQHost ports 33-35, as I use

nodedrv4 0x62 34 2		the first 32 for a PacketCluster Node.

nodedrv4 0x63 35 3		I have given NOS access to 2 radio ports,

nos				and a loopback port to the switch.

This is the 'attach' bit of AUTOEXEC.NET. I can't see where the parameter

following the interface name is used, but the code suggests it is the

maximum length of the transmit queue. The next param is the MTU.

#

attach packet 61 ax1 5 256		Radio Port 1

attach packet 62 ax2 5 256		Radio Port 2

attach packet 63 node 5 236		Loopback link to the switch

#

This is a new facility, and there is very little local IP activity for

me to try it with. Any feedback would be most welcome, both on problems

and suggestions on how best to set up the system.

#####