DLP HS/Link

Copyright 1993-1995 Samuel H. Smith

All Rights Reserved

A version of the HS/Link File Transfer Protocol

for use with

ProComm Plus for Windows Version 2.x

HSLINK.DLP is a special version of the HS/Link program which has been

written specifically for use by ProComm Plus for Windows Version 2.x

(PROWIN2).

INSTALLATION

Simply place the HSLINK.DLP in your PROWIN2 directory, and the HSLINK

protocol will automatically appear in the protocol menus and will work

as other protocols do in this environment.

If you wish to include HS/Link in your "host mode" operations, copy the

supplied _HOST.WAS file to your PROWIN2\ASPECT\HOST directory.

OPERATION

Once HSLINK.DLP is installed in your PROWIN2 directory, the HS/Link

protocol will automatically appear in PROWIN2's protocol menus.

Downloads are automatic and require no special attention.

To upload, click the "upload" icon and select files to transmit.

*** IMPORTANT ***

To perform a bidirectional transfer, note that you

MUST NOT press the final <ENTER> key required by the

BBS you are calling.  Instead, click the "upload" icon

and flag outgoing files.  When HSLINK starts on your

end, it will send the final <ENTER> key sequence to

the BBS.

CONFIGURATION

If you wish to set HS/Link options, you can modify the HSLINK.INI file

in your PROWIN2 directory.  These settings are normally manipulated via

the "Advanced!" setup option of HS/Link from within PROWIN2.

To configure HSLINK.DLP, activate ProComm+ "Current Setup" ICON, push

the Transfer Protocol button, then select HSLINK as the "Current

Protocol".

Crash Recovery settings

Off

Disable crash recovery.  Do not allow received files to

overwrite existing files.  Partial files from aborted

transfers are automatically erased.

Full Recovery

Enable full crash recovery.  This option causes partial

files from aborted transfers to be retained.  If a

received file matches the filename of an existing file,

the contents of the file is compared to the received

file, and only those blocks which differ are

transferred.

Overwrite

Disable crash recovery.  Allow received files to

overwrite existing files.  Partial files from aborted

transfers are retained.

Block Size settings

256     Each block will be 265 bytes in length

1k      Each block will be 1024 bytes in length

2k      Each block will be 2048 bytes in length

4k      Each block will be 4096 bytes in length

Acknowledge Window Settings

Full streaming (none)

In this mode of operation, blocks are transmitted

continuously with no acknowledgement of receipt.  This

mode of operation is useful when error correcting modems

are used or when flow control is not required.

8 blocks (normal)

In this mode of operation, up to 8 blocks may be in

transit before an acknowledgement of receipt is

required.  Once acknowledgement is received, the next

block will be transmitted.  This acknowledgement process

provides a form of flow control which is useful when

other forms of flow control are not totally reliable.

4 blocks (PC Pursuit)

When running HSLINK on a PC Pursuit connection, I

recommend the settings of Ack Window = 4, Block size =

256, Minimal Blocks = OFF.

File Settings

Transfer Log File:

This defines the DSZLOG-style transfer result log

filename.  If left blank, the DSZLOG environment

variable specifies this filename.  If no DSZLOG

environment setting exists, no logfile will be produced.

Error Log File:

This defines the filename for a detailed transfer and

error log to be created.  If left blank, the HSERR

environment variable specifies this filename.  If no

HSERR environment setting exists, no logfile will be

produced.

Duplicate Checklist:

This defines the filename of a duplication checklist

file.  The duplication checklist file can contain a list

of directories or filenames to check for duplication. If

an incoming file matches a filename or a file in a

directory listed here, the file will be rejected.

NOTE: duplication checking is active ONLY when Crash

Recovery is set to "OFF".  Other crash recovery modes

grant HSLINK permission to modify existing files, making

a duplication checklist meaningless.

Other Options

Original Time Stamp

Checking this option causes the original file's time and

date to be preserved.  Unchecking the option causes the

current time and date to be given to a file when it is

received.

Minimal Blocks

Checking this option causes HSLINK to minimize each

block transmitted, resulting in an increase in thruput.

Unchecking this option causes HSLINK to include

additional information in each block as required by PC

Pursuit or some other environments.

File Buffering

Checking this option causes HSLINK to use a 10k buffer

for files being received or transmitted.  Unchecking

this option causes HSLINK to read or write each file

block as it is sent or received.

Allow Bidirectional Transfers

Checking this option allows files to be received while

files are being transmitted.  Unchecking this option

causes HSLINK to reject any incoming files when files

are being transmitted.

Beeper in Chat Mode

Checking this option allows the remote unit to beep your

speaker with the "page remote" or ^G key in CHAT mode.

Unchecking this option forces your speaker to remain

silent at all times.

Force Remote to use Local Settings

Checking this option causes forces the remote unit to

use your settings for Minimal Blocks, Block Size, Ack

Window and Xon/Xoff.  Unchecking this option allows the

remote unit to override your settings, or causes

compromise settings to be negotiated according to both

local and remote options.

Controls

OK

This causes current settings to be saved.

Cancel

This causes changes in settings to be discarded and

previous settings to remian in effect.

About HS/Link

This will display information about this version of HS/Link.

Register...

This allows you to enter your personal registration code

as well as your name and company name.

DIFFERENCES

HSLINK.DLP is similar to the standard DOS HSLINK with the following

exceptions:

*  GIF files are automatically displayed onscreen during file transfer.

*  The command line is automatically provided by PROWIN2.  Use the

integrated setup screens, or modify the HSLINK.INI file to provide

protocol options.

*  The "REGISTER" screen now accepts DLP HS/Link activation codes.

Sorry, but DOS HS/Link registrations are not valid for the Windows

DLP version.