HOW TO SETUP A DOOM NETWORK

---------------------------

Intro:

When considering setting up a DOOM network I found most of the information

available on bulletin boards to be very confusing so after I got one

setup and running I decided to write a .doc file that tells how step-by-step

I did it - if you follow the steps it SHOULD work (no guarantees!).

===============================================================================

Hardware:

First you need Network Interface Cards (NIC's) - these are available in

several different standards (Arcnet and Ethernet being the most common) -

You want Ethernet cards that use RG-58U Coax (10 Base 2 in network terms).

The step-by-step instructions I am presenting are using StorQuest

STQNETI/EC cards (these are the same as EFA InfoExpress S16C).  These are

basic Ethernet cards that are totally software configurable - you should be

able to find these for about $50 or less.  If you can't find these cards

any basic Ethernet card that is NE2000 compatible should work fine, and

hopefully will follow the same installation procedure.

Besides the cards you will need  a "T" connector (usually included with the

cards) for each card, two 50 ohm terminators and enough RG-58U coax with

BNC ends to connect the cards together - all this should be available from

the company you buy your cards from.

===============================================================================

Software:

You will need your NIC driver software (included with the card)

and some support files (listed in the "Installing the software" section)

===============================================================================

Hooking up the hardware:

Install the NIC cards and configure them (this is where the jumperless,

software configurable cards are nice.).

Install a "T" on the back of each NIC.

Hook the RG-58U coax between each card.

Install a 50 ohm terminator on each of the end computers (or both on a two

player network).

Example:

< or >   indicates a terminator

T        indicates a BNC T

===      indicates the cabling

<T===============T==============T===============T>

-----------     -----------    -----------      -----------

Computer #1     Computer #2    Computer #3      Computer #4

===============================================================================

Installing the software:

I have included the required support files with this zip file - these files

are intended for Novell Netware, Netware Lite, or Personal Netware use but,

are freely available from the major BBS systems who have Network conferences.

I would suggest you create a DOOMNET.BAT file to automate the driver

installation.

So far I have found two methods to install the software (I'm sure there's

an NDIS to IPX converter out there somewhere I haven't found yet!):

1) Use ODI to IPX converter software.

Before you want to run a network game do the following:

load LSL.COM

load <card specific ODI driver (see note)> mine is ETH16.COM

load IPXODI.COM

note: My driver disk came with a READ.ME file that listed the directories on

the disk and the drivers they contained - the required driver is in the

A:\NOVELL\WKSTN directory, the description is "Novell Netware V4.0

ODI workstation driver".  Hopefully your driver disk uses the same

format - just look for an ODI workstation driver.

Also note that you may have to add a command line parameter if your

card was assigned an IRQ and Base Address other than the default

by the install software (if you didn't pay attention re-run the

install software and pay attention!)

You are ready to load SETUP and play DOOM (refer to DOOM docs for more info.)!

2) Use Packet to IPX converter software.

Before you want to run a network game do the following:

load <card specific Packet driver (see note)> mine is ETHPD.COM

load IPX.COM

note: My driver was in the A:\PACKET directory, the description is

"FTP PC/TCP Packet Driver".

This driver requires some command line parameters to be passed:

ETHPD  <-d> <-n> <-w> <packet_int_no> <int_level> <io_addr>

<-d, -n, -w>    Ignore these, not used for DOOM

<packet_int_no> is the software interrupt vector number

used by the packet driver program, which

can be from Hex Address 0x60 to 0x80

(the docs say you can use decimal 96 to 128)

(but the driver complains when I do.       )

I have found that 0x60 works for me, if it

doesn't work I have read that 0x7E is a good

bet.  If all else fails try them all from 0x60

to 0x80!

YOU MUST PASS THIS VALUE, THERE IS NO DEFAULT

<int_level>     OPTIONAL, this option is the hardware

interrupt (IRQ) line set on your network

interface card. The default is 3 if this

parameter is not specified.

This is set by the install software, if you

didn't pay attention re-run it and pay attention!

<io_base>       OPTIONAL, this option is base I/O address

set on your network interface card.

The default address of 0x300 is used, if

this parameter is not specified.

This is set by the install software, if you

didn't pay attention re-run it and pay attention!

For Example:  C:>ETHPD  0x60  3  300

|    |   |

|    |  \|/

|   \|/  <io_base>

\|/   <int_level>

<packet_int_no>

You are ready to load SETUP and play DOOM (refer to DOOM docs for more info.)!

===============================================================================

Miscellanious:

Doom V1.2 has a bug - if you abort before a net game starts you may get a

"OPEN SOCKET 0xFF" error, you will have to reboot.

I have also included a file called IPXFER11.ZIP - this file will allow you

to send files from one computer to the other through the network cards (sure

beats "sneaker net" when transferring those add-on WAD files!)

===============================================================================

Conclusion:

I hope this helps you set up your DOOM network - I found the hardware install

to be simple (thanx to jumperless, software configurable NIC's - I highly

recommend these for painless install!).  The software install is where I

had the most difficulty (mostly finding the required support files and

determing which drivers to use) - I have tried to help out here by providing

the support files and explaining which driver files I used and where I found

them on my driver disk.

Have fun!

First (& probably last) release August 30th, 1994.