Kurta WinTab Drivers for Windows -- Version 1.09

Copyright Kurta Corporation 1993.  All Rights Reserved.

This file contains installation and setup instructions for Windows 3.1.

If you have any questions please contact Kurta Technical Support at

(602) 276-5533.

--------------------------------------------------------------------------

INSTALLATION

--------------------------------------------------------------------------

* Overview

There are 3 parts to the installation:

1. Install Wintab

2. Install a Device Driver

3. Configure the Tablet Manager

(Details directly below)

* Procedure

1. Install WinTab:

Copy WINTAB.DLL and K_TM.EXE from the WinTab driver disk or directory

(e.g. a:\ or c:\wintab) to your WINDOWS directory.

2. Install a Device Driver:

It is possible to run your tablet in Windows as either the only pointing

device, or in conjunction with a mouse that is plugged into another port.

If you choose to run in conjunction with a mouse, then within any

application, you are automatically able to switch back and forth between

using either device. Decide which way you wish to run, then follow the

appropriate section A. or B. below (BUT NOT BOTH).

A. Installing Tablet as the Only Pointing Device:

Go to the Windows directory in DOS (do not start Windows) and type "setup".

Highlight the "Mouse" line under "System Information" and press return.

Choose "Other" from the list that appears.  Then enter the path of the WinTab

driver disk or directory.  Follow the screen prompts to save your new

selection.

B. Installing Tablet in Conjunction with a Mouse:

From within Windows, open the Control Panel, then open the "Drivers"

Applet. Select "Add", then choose "Unlisted or Upgraded Driver" from the

list that appears.  Indicate the path of the WinTab driver disk when

prompted. Once the Wintab driver has been added, exit Windows and go to the

Windows directory in DOS. Type SETUP. If the driver for your mouse is not

already indicated here (on "Mouse" line under "System Information"), then

install the appropriate mouse driver here now.

3. Configure the Tablet Manager:

Start WINDOWS.  From the PROGRAM or FILE Manager, type ALT-F R K_TM (and

press ENTER). This should bring up the Tablet Manager.  If it does not, type

Ctrl-Esc on the keyboard to bring up a Task List, and switch to the Tablet

Manager from there. Once in the Tablet Manager, type ALT-S H to pull down the

setup menu and bring up the Hardware Setup.  Use the TAB and Cursor Keys

to setup the appropriate tablet type, size, resolution, com port, and baud

rate.

* If you have "Windows for Pen Computing"

Use the Drivers Applet in the Windows Control Panel to install the WinTab Pen

Interface, WTPEN.DRV.  This pen driver makes any WinTab-compatible pointing

device pen-compatible.

--------------------------------------------------------------------------

Trouble Shooting Tips

--------------------------------------------------------------------------

Before proceding to Troubleshooting, make sure that all the steps of the

Installation Procedure above have been completed correctly.

* Driver Conflicts

If you have two tablet drivers installed at the same time, they will conflict

generally causing the tablet not to respond. To check for this problem, first

start Windows. Then open the Windows Setup icon and note the driver on the

"Mouse" line.  Close this icon, then check the "Installed Drivers" in the

"Drivers" applet in the Control Panel.  Make sure that you do NOT have a

tablet driver in BOTH places.

* 386 Enhanced Mode Virtual Device Drivers

If you are having trouble getting pointing to work in 386 Enhanced mode, you

may need to set the virtual device driver com port profile strings manually.

If you have installed for the tablet only, without a mouse, add the following

lines to the end of the SYSTEM.INI file:

[VirtualTablet]

port=<X>

where X is a number from 1 through 4 indicating ports COM1 through COM4.

If you are have installed for the tablet AND a mouse, at the end of the

SYSTEM.INI, add the lines:

[vsertd.386]

WTKurta=<X>

* Combase

You may need to look at this item if either:

A. There is no tablet response in Windows.  OR

B. The tablet works fine in Windows and DOS mouse programs, but there is no

tablet response when you are shelled out of Windows to a DOS mouse program.

In your System.ini file in the Windows directory, in the [386 Enh] section,

look for the Combase lines.  If your tablet is plugged into Com 1, and a

line that begins "Com1base" exists, then remove this line .  If your tablet

is plugged into Com 2, and a line that begins "Com2base" exists, then remove

this line.