�LM0��RM255�

DIAL.PM v2.0i for XyWrite III v3.1x, III+, and Nota Bene Word Processors

=======

Purpose

=======

Lift phone numbers from any document, e.g. an address book, and dial voice

calls with an internal or external modem.

===========================================================================

QUICK START:  Dedicate an ordinary Save/Get A-Z or 0-9 in <<SV01>>,

SAve DIAL.PM, and then RUN DIAL.PM.  But be advised, this document contains

IMPORTANT information about DIAL.PM:  read it!

===========================================================================

DIAL.PM operates at several levels of complexity.  At its simplest, it

grabs any phone number off a document, and dials that number (or dials a

number that you enter manually) through your modem.  With properly formatted

documents, DIAL.PM accesses prefixes automatically.  Formatting takes a bit of

effort, but it pays off in ease of use.  The pleasure increases with the size

of your address book and the length of numbers dialed (especially

international numbers).

On No-Answer or a Busy-Signal, at your option DIAL.PM will re-dial

automatically and continuously until you connect.

DIAL.PM places the dial-string in the program's reserved (configurable)

Save/Get, so that you can re-dial anytime after DIAL.PM quits, simply by

keying that Save/Get.  (See also update notes for v1.2a, at end, for use of

extended Save/Gets with XyWrite III+ v3.54+.)

DIAL.PM requires one external file, ADT.COM, which raises or lowers DTR

(`Data Terminal Ready') on the communications port, or sends Hayes-type `AT'

commands to a modem.  ADT also functions independently, as a modem controller;

type ADT<cr> from DOS, for a command summary.  (ADT is based on AT.COM and

DTR.COM, written and placed in the public domain by Donavon Kuhn and

Jon Niedfeldt [DOSMODEM.ARC].)

For XYW-DIAL v1.1+ changes, see LaBels SV01 and SV10, below, and the update

summary at the end of this document.  There have been fixes and a few changes.

Specifying Phone Numbers

========================

If numbers to dial are FULLY stated in the document, then no special

formatting is required.  In other words, if the numbers are local, or all

necessary access codes are written out in entirety, simply place the cursor on

any digit in the number, and RUN DIAL.PM.  Hit <cr> (the Enter key) at the

first program prompt to dial the number.  (Read `Install DIAL.PM', below,

before you try this.)

The following telephone-number formats are legal:

1234567              pure numbers

123-4567             dashes (Ascii 45) are ignored

212-123-4567         area codes

1-212-123-4567       phone company access codes

91-212-123-4567      get an outside line with e.g. `9'; dial the number

9,1-212-123-4567     insert pauses with commas (wait for dial tone)

^                     **each comma = a 2 second pause (Hayes standard)**

011-62-21-1234567    AT&T international call (011) to Indonesia (62)

Jakarta city (21) local number 1234567

011-62-{0}21-1234567 same as above, but preserve a note in your XyWrite

^^^             text document that if you are IN Indonesia and making

a trunk call to Jakarta, you will need to prefix the

city [area] code 21 for Jakarta with a zero

^^^             **braced numbers {nn}, e.g. {0}, are ignored**

9,,01162{0}211234567 get an outside line, then same as above

Thus, digits 0123456789, commas, dashes, and PAIRED {} braces, are LEGAL.

ALL other characters, such as ()/\|.;:[]_ and SPACES Ascii 32, are ILLEGAL.

Trailing commas are discarded.

When the program scans a phone number and encounters an illegal character, it

interprets that character as the beginning or ending boundary of the legal

dial-string.

Formatting Prefixes in Documents

================================

A special PRompt format allows you to collect all numbers within a domestic

Area Code, foreign country, or foreign city, in grouped listings.  The special

PRompt is <<PRTL>> followed immediately by the prefixes which apply to the

entire group ( <<=Ascii 174 and >>=Ascii 175 ).  Place this PRompt in your

document from the CoMmand line:  PR TL<cr>.  Prefixes are not stored INSIDE

the PRompt, but immediately following it:

<<PRTL1-301>> is wrong, <<PRTL>>1-301 is correct.

Excerpts from an address book:

Baltimore: <<PRTL>>1-301

---------

Baltimore Museum of Art, Art Museum Drive, 21218, 396-7101

London: <<PRTL>>011-44-{0}1

------

Museum of Mankind, 6 Burlington Gdns, W1X 2EX, 437-2220

Victoria & Albert Museum, South Kensington, SW7 2RL, 589-6371

New York City: <<PRTL>>1-212

-------------

American Museum of Natural History, 873-1300

Fire Emergency 628-2900

Metropolitan Museum of Art, 879-5500

Time-of-Day 976-1616

Yemen: <<PRTL>>011-967

-----

National Museum, POB 2606, Sana'a, Yemen Arab Republic, {0}9672-27-1840

To DIAL any of the above numbers, place your cursor on the LOCAL number (not

the <<PRTL>> regional prefix), hit the key which you've assigned to

RUN DIAL.PM, then hit letter `P' (for Prefixes) twice, as prompted by the

program (see the `P'refix dialing command, below).

Install DIAL.PM

===============

The default communications setting is COM1:2400,N,8,1 (serial COM port 1,

2400 baud, No parity, 8 databits, 1 stopbit).  Reset your serial port if your

modem's communications port or baud rate is different:  e.g., from DOS,

command MODE COM2:1200,N,8,1<cr>, where the modem uses COM2: and the baud

rate is 1200.  As a practical matter, put this MODE statement in AUTOEXEC.BAT.

If COM2:, COM3:, or COM4: is the modem's serial port, then modify DIAL.PM at

variable SV03 (see `<<Sv03>> Serial_COM#:_Port', below).

Extract DIAL.PM and ADT.COM from XYW-DIAL.ZIP, and locate these files together

in the usual XyWrite directory.  (Actually, DIAL.PM may be located anywhere IF

the key which RUNs DIAL.PM has a proper Path statement - see next paragraph.

ADT.COM can be located elsewhere IF its Path is stated in variable SV02 - see

`<<Sv02>> Drive:\Path\_for_ADT.COM', below.  The default for both files is the

current directory.)

For convenience, put DIAL.PM on a key in your KeyBoarD file, e.g. Control-P:

Call the .KBD file, go to TABLE=CTRL, key #25 (=P), and enter:

25=BC,r,u,n, ,c,:,\,x,y,\,d,i,a,l,.,p,m,XC

recommended:  >> ^^^^^^^^^^^^ <<  include Path to DIAL.PM

Reload the keyboard, e.g. ldkbd ibm.kbd<cr>.

DIAL.PM requires the use of one normal Save/Get key (A-Z, 0-9) to function;

indicate the key you wish to use in <<SV01>>.

N.B.:  A key **MUST** be indicated in <<SV01>> before DIAL.PM can be used!

--------------------------------------------------------------------------

There are ten hard-coded variables within DIAL.PM.  CAll DIAL.PM onto the

screen, enter expanded editing mode, and note the options displayed at the

top of the program:

<<Sv01>> Save/Get_key_for_DIAL.PM_(no_key_defined=�AESV01,�AF=UNINSTALLED)

This is DIAL.PM's hard-coded, "reserved" Save/Get.  See also update notes to

v1.2a, at end of this doc.

<<Sv02>> Drive:\Path\_for_ADT.COM_(default=none=current_dir)

By default, ADT.COM is assumed to be located in the current drive and

directory.  It would be prudent to indicate here, in <<SV02>>, the drive and

directory where ADT.COM is physically located, so that DIAL.PM can always

find it.  Change the default <<SV02,>> to, e.g., <<SV02,C:\XY\>> where C: is

ADT's drive and \XY is its directory.

<<Sv03>> Serial_COM#:_Port_(`#'=1|2|3|4)_(default=1)

Serial port COM1: is the default.  Change the port-number in <<SV03,1>> to

2, 3, or 4 as required by your hardware.                            ^

<<Sx04>> Wait_to_Disconnect_(default=15_seconds)

Indicate the number of seconds to wait (keep ringing) for an answer.

<<SX04>> controls Repeat-dial delay also.  It manipulates the modem's S7

register.  The default interval between redial attempts is 15 seconds (plus

3-4 seconds for ADT loading and dialing).  You can alter this value for

any single run from the main prompt (the `D'elay switch); change the default

permanently in <<SX04>>.

<<Sv05>> Additional_Modem_Commands_(default=none)

If you regularly give the modem additional `AT' commands, enter them here.

Do not use the characters `AT' but only enter the commands, e.g. M2 (speaker

always ON), S8=1 (change the duration of a comma pause to 1 sec from

default=2), Z (reset; some modems freeze on this), or X3 thru X6 (disconnect

on busy signal; some modems only).  Combine commands, e.g. <<SV05,ZM0S8=1>>.

Don't try to manipulate register S7; DIAL.PM does that with <<SX04>>, above.

<<Sv06>> `T'one_or_`P'ulse_Dialing_(default=T)

Change to <<SV06,P>> if your phone doesn't support Touch-tone dialing.

<<Sx07>> Machine_Speed_Factor_(default=65)

(Repeat-dial only.)  This variable controls the program's software timer,

which is dependent in turn on the computer's speed.

*** HOW TO CALIBRATE SPEED FACTOR: ***

-----------------------------

a CAll DIAL.PM to the screen, enter expanded editing mode, and find

LaBel_VARIABLES at the top of the file.

b First, test the setting for variable <<SX08>>:  RUN DIAL.PM, hit `M' for

Manual dial, and type the number of your own phone <cr>.  This generates a

Busy signal.  Listen in and time how long it takes, AFTER the ADT commands

are issued, to get a dial tone and send the beeps or pulses of the

dial-string.  Set <<SX08>>.  Default=3 seconds

c Now RUN DIAL.PM again, hit `R' for Redial, `M' for Manual dial, dial your

phone, then monitor the modem speaker (or listen in on the handset, a click

occurs when the modem goes off-line/disconnects).  Ideally, the modem should

hang-up (go off-line) just before the next Redial is attempted.  If software

Redial occurs while the modem is still on-line, then INCREASE the value of

<<SX07>> step-by-step.  If the modem goes off-line more than a second or two

before Redial, REDUCE the value of <<SX07>>.

d If you've done everything right, different Delay values (e.g. 5 and 20

seconds -- `D'elay is changed at the main program prompt) should result in

the same 1-2 second lag between a modem hang-up and software redial.

17 is an approximate speed factor for a 4.77Mhz PC.

65 (the default) is appropriate for an 8Mhz AT.

90 is approximate for a 10Mhz AT.

Faster machines require higher Speed Factors.

See also <<SV10>>, below.

<<Sx08>> Average_`Get-Dial-Tone_and_Dial'_Delay_(default=3_seconds)

(Repeat-dial only.)  Indicate the average time in seconds that it takes the

modem to get a dial tone and send the beeps or pulses of a typical

dial-string.  See instructions for <<SX07>>, above.

<<Sx09>> Modem_Comma_Pause_Duration_(default=2_seconds)

(Repeat-dial only.)  For most modems, 2 seconds is the default.  This could

be altered with an `AT' S8=# command, issued in <<SV05>> above.  Change

this value to agree with an S8 command routinely issued, or any non-standard

comma pause duration.

<<Sv10>> 1.5-second_Pause_After_`AT Z'_Reset_(Y|N)_(default=`N'o)

An `AT Z' complete modem Reset was added in v1.1.  It occurs just prior to

dialing.  If your modem cannot digest the Reset command in time to dial,

set the SV10 variable to `Y'es.  This inserts a 1.5-second pause (time

actually regulated by <<SX07>>) between `AT Z' and the dial command.

In repeat-dial operation, `AT Z' is issued only before the first dial

attempt.  The pause occurs with Save/Get redial too (if <<SV10,Y>>).

Run DIAL.PM

===========

In v2.0e, a new command-line argument is added, ",D".  This enables

auto-dialing of the number at the cursor location, and bypasses the menu.

The format is:  "RUN DIAL.PM,D".  (Good for XPL programming!)

The available commands appear on the CoMmand line and on the PRompt line:

CM Delay=15|Hangup|Repeat-dial|Prefixes|Txt|manual ###. D|H|R|P|T|#|cr

PR `Esc'ape aborts

^^[XyWrite PRompts]

Optional Commands Issued BEFORE Dialing:

---------------------------------------

`Esc'ape (abort) ends the program from ANY prompt.

`D'elay resets the number of seconds during which the modem rings a number

before the modem hangs-up (current run only; permanently change the delay

duration as described above under `SX04_Wait_to_Disconnect').  The current

setting is always listed on the prompt line, i.e. `Delay=15' (seconds).  For

some international calls, the delay will need to be increased (to 25-30

seconds or more) if you are using the Repeat-dial facility.  If you pick up

the handset before the delay period ends, the phone will continue to ring

even if nobody has yet answered; otherwise the modem will automatically

disconnect the line (hangup).

`R'epeat-dial is for busy numbers.  (You can `C'ancel repeat-dial if you

change your mind, before dialing the number.)  Repeat-dial operates in

foreground only; you cannot use the computer.  A modem speaker is necessary,

to monitor whether you're getting a ringer (if you monitor the handset, the

program's Hangup instruction to the modem is overridden, and redial fails).

When you do get a ringer or an answer, you must pick up the handset and issue

a Break command to the program (function call BK, usually assigned to key

Ctrl-Break).  Break also interrupts and terminates Repeat-dialing anytime.

`T' briefly locates the cursor at the current Text location, then restores

it to the Header.  Double-check that your cursor is located on a legal

dial-string.

Dialing Commands:

----------------

Manual dialing is initiated from the main menu by simply typing the first

digit of a number; enter the complete dial-string, then <cr> to dial.

`P'refix codes offers two options, either manual entry of codes which will be

prefixed to the number that appears at the cursor position in the document

on-screen, or else -- if one exists -- automatic retrieval and prefixing of

the last-previous <<PRTL>> code which occurs in the document, PLUS the number

at the cursor position.  For use of <<PRTL>> formats, see `Formatting Prefixes

in Documents', above.  Hit `P' again to use the <<PRTL>> prefix code and dial.

Any other key, e.g. `1', becomes the first digit in a manually-input prefix.

`T'xt checks the cursor position in-text.

<cr>, the DEFAULT COMMAND, simply dials the number at the cursor position.

*****  ALWAYS Pick up the handset when you get a ringer   *****

*****  or answer, and BEFORE the Delay period expires;    *****

*****  otherwise the modem will automatically disconnect. *****

Function BK [usually mapped to Ctrl-Break].  The XyWrite Break command

(func BK) will stop program execution at any point.  XyWrite prompts:

`Stop Program? (Y/N)'.  Hit Y(es).

`H'angup.  There are several ways to STOP dialing which is in-process or

ringing, and hang-up the modem forthwith.  You can RUN DIAL.PM again (hit

the DIAL.PM `run' key), and issue a `H'angup command (or, if Repeat-dialing,

hit Ctrl-Break and Stop the program first).  From DOS, command

`ADT A 1[-4]'<cr> does the same thing.  The Big Red Switch on an external

modem works too [kill the power].

On termination, DIAL.PM leaves a complete command to dial the last number in

the program's unique Save/Get key.  Thus, simply hitting this Save/Get key

re-dials the number.  The last-used dial-command will remain in the Save/Get

key, and may be used repeatedly, until it is overwritten by DIAL.PM or the

Save/Get key is otherwise wiped out (e.g. with CLRSGT or CLRASG).

DIAL.PM has been tested on XyWrite versions 3.15, 3.53, 3.54, and 3.55 under

MS-DOS v2.11 and IBM-DOS v3.1 and v4.0.

Troubleshooting:

---------------

DIAL won't work!  What's wrong?

First, did you set the baud rate?  2400 baud is the default; if your modem

functions at 2400 baud, you need do nothing, but any other baud rate requires

a MODE command, e.g. MODE COM2:1200,N,8,1 for 1200 baud on COM2.  Issue this

command from DOS:

MODE COMn:bbbb,N,8,1<cr>

where n is the serial com port Number and bbbb represents the Baud rate.

Next, quit XyWrite and go to DOS.  Move to the directory where ADT.COM is

located.  If you have an external modem, watch the lights; if internal,

load a TSR light simulator (available on any BBS, in the Communications

directory).  Command ADT D n OF<cr>, where n is the COM port number; this

turns OFf Data Terminal Ready.  Now command ADT D n ON<cr>; this raises

DTR, and the light should respond accordingly; if not, check your cables.

Have you got a crummy modem that sets DTR always high?  No matter; proceed.

Once you've got DTR up, command ADT A n Z<cr>; this is a modem reset.  On most

modems, both the Receive Data and Send Data lights will flash.  Now command

ADT A n Dt1234567<cr>, where t is "T" or "P" for Touch-Tone or Pulse dialing,

and for 1234567 substitute your own phone number (dial your own phone!).  If

this doesn't work but the preceeding ADT commands were successful, go to your

ordinary communications package and find out what initialization strings your

terminal program sends to the modem.  Suppose the strings are

ATE0, ATQ0, ATV1, ATX6, ATS7=032, ATS9=010, and ATS10=020 (whew!).  Copy them

into a single ADT command as follows (skip all AT prefixes):

ADT A n E0Q0V1X6S9=010S10=020Dt1234567<cr> -- be SURE to SKIP ANY S7 STRING!

Note that the actual DT|DP dial command and dial number are the LAST commands

issued in the string.  OMIT any Z reset command (very important).  Listen in

on your handset; if this works, then repeatedly issue a reset (ADT A n Z),

pause a moment, then one-by-one eliminate as many of the commands as possible

(unless you desire them) to get a barebones initialization string that

works -- and finally write this init string in <<SV05>> (excluding the DT/DP

dial string).

If the initialization string that worked with ADT doesn't work with DIAL.PM,

set <<SV10,Y>> to insert a 1 1/2 second pause between reset and dial commands;

this solves any remaining problem 99% of the time.

Enjoy!

DIAL.PM code may be renumbered with RELOCATE.PM

RELOCATE "Special (Additional) Codes" for DIAL.PM (see RELOCATE.DOC in

RELOCATE.ZIP):

--------------------------------------------------

FREEZE STRINGS:  For Use by RELOCATE.PM

====== =======   (Lowest S/G number=01 in the original version;

SX00             if you have RELOCATEd the code, see the

IS00             program itself for the current low S/G number)

SPECIAL RELOCATE STRINGS:

======= ======== =======

r=

sx�MDRV�W�MDNM�,  ("sv" + function �MDBO�WW�MDNM� + comma). Func WW is often mapped on Ctrl-Alt-W.

Is

Sv

Sx

--------------------------------------------------

Main Version History:

==== ======= =======

v2.0i - 18 Jul 90

Corrects the internal timer for "Repeat-dial" with Pulse phones.  An _average_

rate of 5 pulses per second is assumed, which includes the interval between

digits in dialed numbers (and therefore differs from the rate specified in

your modem manual).  No explicit "User Variable" controls this rate, but it

can be manipulated by SEarching for text string "/5" in DIAL.PM:  raise

the rate above 5 for quicker timeouts, or reduce it for longer timeouts.  The

routine (skipped unless required by "DP" and "repeat-dial") to count the

total number of pulses in the dial string is verbose and necessarily clumsy,

to maintain compatibility with old XyWrite versions (commas are stripped,

and the dial string is parsed, on the CMline).

v2.0h - 4 Jun 90

Significantly faster, tighter, 25% smaller code

Auto-installation routine:  Removed

Enable correction of manually-entered prefixes|numbers [accepts

funcs BD|RC|CL|LL|CR|LR]

Numeric order of user variables changed:  SV10 becomes SV01;

SV01-09 incremented by one (to become SV02-SV10)

Any Save/Get 01-10 setting may be overridden from the keyboard file or command

line, by including an argument in the RUN command-string.  E.g., to address

COM2 (if e.g. the hard-coded COM assignment is �AESV03,1�AF), command:

run dial.pm,�AESV03,2�AF�FF��<cr> -- the last 3 chars are "{Ascii-255)��"

or from the .KBD file:

nn=BC,r,u,n, ,d,i,a,l,.,p,m,CO,�AE,S,V,0,3,CO,2,�AF,�FF,�,�,XC -- using 3-byte

forms of "�AE", "�AF", and {Ascii-255)

v2.0g - 18 Oct 89

Simpler code

v2.0e - 27 Aug 89

Auto-dial at the cursor position (no menu) by using the command-line

argument ",D" (e.g. "run dial.pm,d").  Among other things, ",D" enables use

of DIAL.PM within other programs that locate the cursor on a dial string.

DIAL must be configured by the user in advance, to properly dial.

v2.0d - 23 Aug 89

Fixed crash caused by Error Help VAriable EH=1 (Error Help disabled during run

[EH=0] then restored to original value on termination)

v2.0c - 22 Aug 89

All VAriables changed to $VAriables, where acceptable, so that current value

rather than default value is read

v2.0b - 29 Jul 89

fixes for XyWrite v3.55

v2.0a - 22 Jun 89

revised DIAL.DOC

minor fixes/changes

ZIPped

v1.2c - 17 Mar 89

eliminated <<EX1>>-type exit

simplified code

v1.2a - 20 Feb 89

For XyWrite III+ v3.54 and higher ONLY:  DIAL.PM's "reserved" Save/Get key

can be any extended Save/Get in the range SV44-SV999 (no ordinary

Save/Get A-Z, 0-9 need be reserved, though one can of course be used, and

most users will want them for redial simplicity).  Note that an

extended Save/Get MUST be HARD-CODED by the user in the program at variable

SV01 (e.g. <<SV01,44>> ).  If you use RELOCATE.PM to renumber the Save/Gets

in DIAL.PM, the hard-coded Save/Get number in SV01 must be reentered

manually.  Save/Gets SV100-SV999 remain in the Save/Get buffer

and can be accessed after DIAL.PM terminates for redial purposes, whereas

Save/Get's in the range SV44-SV99 do not remain in the buffer and

therefore DISABLE REDIAL.  With Save/Gets SV100-SV999, you can redial with

any keystroke combination, by creating a small program named e.g.

REDIAL.PM.  If for example you select Save/Get 101, code REDIAL.PM as

follows:

<<PV101>><<EX>>       [N.B.: "<<"=Ascii 174 and ">>"=Ascii 175]

Then CAll your .KBD keyboard file and load REDIAL.PM on any key

combination, e.g.:

BC,r,u,n, ,c,:,\,x,y,\,r,e,d,i,a,l,.,p,m,XC

For the sake of speed, there is no crash protection if you use an extended

Save/Get but your XyWrite version is lower than v3.54.  Type "help<cr>" on

the CoMmand line, to obtain your Version number.

much added error-proofing

changed `A'bort key to Escape key

`T'ext command: check current cursor location

simpler entry of manually-dialed numbers and prefixes

display the current <<PRTL>> Prefix, if any, on the Prefix sub-menu command

line

simplified Installation

abort Installation with Escape

DIAL.PM Save/Get key can be hard-coded in <<SV01>>, or installed

automatically by the program

v1.1c - 22 Dec 88

unload keyboard buffer on termination

v1.1b - 27 Nov 88

cosmetic changes

v1.1a - 14 Sep 88

inserted `AT Z' modem Reset (between DTR `ON' and `AT D' Dialing)

added user-variable SV10: enable|disable 1.5-second Pause After `AT Z'

Reset, for fast computers/slow modems (default: disabled)

slightly smaller size, slimmer routines, faster operation, clearer PRompts

corrected redial-timer math for commas in dial-string

removed CLRXSGT command from install routine

converted all AT strings to UPPER CASE, for lousy modems

simplified manual input of dial prefixes (one less keystroke)

couldn't re-install Save/Get `0': fixed

crashed on Delay=nul: Delay=nul disallowed

auto-redialing aborted on Ctrl-Break only at end of Delay period: fixed,

now enabled anytime

accepted illegal Save/Get install keys: fixed

accepted dial-string if prefix good despite No number at cursor: fixed

DIAL.PM v2.0i for XyWrite III(+) and Nota Bene

R. J. Holmgren, 410 Central Park West, NYC 10025

Messages: Any RelayNet BBS carrying the XyWrite Conference;

or Word Processing BBS (708) 491-6995

18 July 1990

Modify DIAL.PM as you see fit.  Tell me if it works for you.

Please distribute XYW-DIAL.ZIP intact.

If you like this program, try XYW-DSZ.PM (available in XYW-DSZ.ZIP),

for transferring (up|download) files between two XyWrite users over

voice lines.  Zmodem protocol is used for faultless transfers.