============================================================================

DALLAS REMOTE IMAGING GROUP BBS    =====>>  214-394-7438

FAX             214-492-7747

VOICE             214-394-7325

DEDICATED TO SATELLITE IMAGERY, SATELLITE TRACKING, NASA,

DIGITAL SIGNAL PROCESSING, IMAGE PROCESSING,

AMATEUR RADIO, AND ELECTRONIC INTELLIGENCE

PCBoard SUPPORTS 6 Lines (USR Dual Standard V.32bis V.42 24 hrs. Daily)

E-MAIL FOR Weather Satellite Report and InterNet UUCP News Groups

ON-LINE HAMCALL APPLICATION - LOOKUP HAM CALLSIGNS

On-Line SATTRAK Satellite Tracking Routines

On-Line SHOP Satellite Resource Center - On-line Ordering

On-Line BALLTRAK  Balloon Launch and Tracking Program

THOUSANDS OF IMAGE FILES OF SATELLITE IMAGE AND NASA VOYAGER IMAGES

SATELLITE TRACKING PROGRAMS, IMAGE PROCESSING, DSP, ATV, SSTV, FAX

============================================================================

Documentation for SBDSP - SoundBlaster APT Demodulation Program

----------------------------

This program and code are copyrighted material of the Dallas Remote Imaging

Group, and may NOT be copied in any fashion without the written consent of

the Dallas Remote Imaging Group.

Introduction:

The SoundBlaster and SoundBlaster Pro audio I/O cards from Creative

Labs, Inc.  are very popular accessories found in many IBM PC compatible

computer systems.  Although the boards are usually used only to provide

music and sound output for computer games, they also support audio input

into the computer.  The standard SoundBlaster supports mono audio I/O

and is available in the $100 price range.  The Pro model supports stereo

and adds a CD/ROM (SCSI) interface.  It costs around $250 dollars.

The boards can digitize sound at high speed and, using one of the

computer's DMA (direct memory access) channels, transfer the sound

samples directly into computer memory.	Once in memory the sound samples

are written to disk.  It is this ability to digitize and record sound

that enables the SoundBlasters to be used for the reception and decoding

of APT satellite signals.

The output of a radio receiver tuned to an APT signal is just a 2400 Hz

audio tone.  As the satellite scans the earth and clouds,  it varies the

amplitude of the tone in proportion the the brightness of the area that

it is currently observing.

-------

What it does:

If you viewed the contents of the .VOC sample file produced by the

SoundBlaster VREC program with your favorite image processing program,

it would not look at all like a weather satellite photo.  The .VOC file

is like an exposed roll of film.  Its contents must first be "developed"

before you can view the image.	The process of "devloping" the .VOC

file contents into a weather satellite image is called DEMODULATION.

Demodulation converts the recorded values of the samples of the 2400 Hz

APT sine wave into their corresponding pixel values.  The desired image

pixel values actually represent the amplitude of each peak in the

APT sine wave signal.

Signal sampling theory states that if you are to do anything meaningful

with a digitally sampled signal then you must sample the signal at least

twice as fast as the highest frequency component of the signal.  The

2400 Hz APT signal needs to be sampled at least 4800 times a second.

Because the APT signal is not a constant tone,	it has freqency

components much beyind 2400 Hz.  Also the laws of thermodynamics (and

life) say that nothing is perfect as theory predicts and you really need

to sample at a higher rate.

If we sample the APT signal at exactly 4 times its 2400 Hz

carrier frequency (i.e at 9600 samples per second), each input sample

will be eactly 90 degrees (360/4) in phase apart from its adjacent

samples.  Using the following trigonometry identities it turns out to be

particularly easy to accurately demodulate the APT samples:

cos(x) = sin(90-x)

sin(x)*sin(x) + cos(x)*cos(x) = 1

By substituting the first equation into the last equation we get:

sin(x)*sin(x) + sin(90-x)*sin(90-x) = 1

Because each recorded sample of the 2400 Hz APT sine wave is 90

degrees apart from the next sample we can calculate the pixel value as:

pixel = sqrt((sample[t]*sample[t]) + (sample[t-1]*sample[t-1]))

The demodulation program just needs to read the .VOC sample file, throw

away the file header information,  adjust the samples for clock/DMA

rate errors and DC offsets, calculate the pixel values from adjacent

pairs of sample values,  scale the results to values from 0 to 255, and

finally write the resulting pixel values to the image file.

This could be a very time consuming process when you consider that a

typical 14 minute capture has about 8 million input samples and that

multiples, adds, and square roots are all operations that take lots

of CPU time.  This is particularly true if floating point arithmetic is

used.

To make the demodulation program run very quickly a table of all 65536

possible combinations of adjacent 8-bit sample values is pre-computed

according to the above formula.  All the demodulation software then has

to do in order to convert the recorded APT samples into their pixel

values is look up the proper table entry.  No time consuming floating

point arithmetic need be used once the demodulator table has been built.

Reading and writing 8 megabyte files is always a time consuming process.

By always reading and writing the sample and image files in multiples of

512 bytes, the demodulation software takes full advantage of the

behavior of the DOS file system.

-------

Problems,  problems,  problems:

Although the SoundBlaster is quite good at adding a little zip to the

odd computer game,  it is far from being a precision signal processing

system.   Using the SoundBlaster card for APT reception presents many

problems.  Fortunately software techniques may be used to overcome

the problems and yield APT images that rival the best commercial

systems.

Using the SoundBlaster for APT demodulation presents two main types

of problems:

1) Sample inaccuacies and offsets

2) Clock rate and missing sample errors

Sampling a perfect sine wave with a perfect board in a perfect world

should yield a series of samples with an average sample value of zero.

Unfortunately the SoundBlaster circuitry usually adds a small DC offset

to the recorded sample values.	This DC offset can cause noise and

"beat" patterns to appear in the demodulated image file.

To remove the effects of any DC offset in the sample values, the

demodulation software calculates the average sample value in each block

of input samples.  This sample bias is then subtracted from each input

sample.

The demodulation program is currently a bit paranoid about things and

re-calculates the sample bias for each input block.  It would be a

bit faster to calculate the value for only the first block.  If the

input level to the SoundBlaster board is properly adjusted, the sample

value offset does not seem to change between blocks.

For easy demodulation of the APT signal we need to sample at 9600

samples per second.  If the resulting image is to be properly

synchronized to the satellite data this needs to be EXACTLY 9600 samples

per second.  PRECISELY EXACTLY 9600 samples per second.  How exact?

A typical fourteen minute satellite pass requires an accuracy of around

one part per 10 million...  9600.0000 samples per second.

Typically a satellite scans one line of pixels every 1/2 second (120

lines per minute).  At a sample rate of 9600 samples per second, a 14

minute satellite pass generates about 8 megabytes of data.  If the

sample clock rate is only accurate to a mere one part per million, the

resulting image will be skewed 8 pixels from top to bottom.  Although 8

pixels of skew across of screen of 4800 pixel lines is only about a .1

percent skew, it is noticeable if you are looking for it or are wanting

to do some quantitative analysis of the satellite orbit from the doppler

bow.

How accurate is the SoundBlaster clock you ask?  It turns out the

closest that you can set the sample rate is around 9620 samples per

second...  an accuracy of around one part in a few hundred...  not even

close.	To make matters worse the clocks on any two SoundBlaster boards

can easily differ by parts per thousand.  Clock errors of this magnitude

cause the image to appear like a TV set that has lost sync.  The image

is an unreadable mess of slanted bars.

The DMA circuitry on the SoundBlaster boards can automatically transfer

blocks of up to 32768 signal samples into the computer's memory without

using any CPU time.  Once a block of samples has been DMAed into memory,

the recording software starts DMAing the next block of samples into

another memory area.  While the second block of samples is being

received, the recording software writes the first block to the

computer's hard disk.  The process alternates in a ping-pong fashion

between reading samples into one buffer while writing the other buffer's

data to disk.

Unfortunately a few signal samples are lost in the time between the end

of one DMA block and the startup of the next DMA block.  To make matters

worse the number of samples lost depends on the computer system's CPU

type and clock rate.  Faster systems loose fewer pixels between blocks.

Also number of pixels lost seems to depend upon whether the block is an

ODD or EVEN numbered block.

Fortunately it is possible to compensate for the errors in the clock

rate and the samples lost between DMA blocks.  The demodulation program

compensates for the pixels lost betwwen DMA blocks by duplicating the

last few samples of each DMA block.  The user can independently specify

the number of samples to add to (or even remove from) the odd and even

numbered DMA blocks with the /O and /E command line parameters.

The sample clock rate error is compensated for in two stages.  The first

stage is the coarse adjustment factor.	This factor causes every n'th

input sample to be removed (or duplicated).  By throwing away

every 601'st sample, the 9620 samples per second that the SoundBlaster

produces is cut down to ABOUT 9600 samples per second.	The coarse

adjustment factor can be changed by using the /C command line parameter.

The second clock rate adjustment factor is the fine tuning factor.  It

is used to zero in the exact timing value.  It adjusts out any

residual skew in the image caused by variations in individual

SoundBlaster board clock generators.  The fine adjustment factor

works just like the coarse adjustment factor.  It causes every n'th

input sample to be removed (or duplicated).  It may be specified with

the /F command line parameter.

-----------------------------

Doing it:

The first step is to connect your satellite receiver to the microphone

input of the SoundBlaster.  You must be very careful to set the

amplitude of the receiver output low enough so that it does not

overdrive the SoundBlaster.  If you overdrive the card its automatic

gain control (AGC) circuitry will attempt to compensate.  This will

cause streaking in the resulting images.

To properly set the receiver level use the VEDIT2 program supplied with

the SoundBlaster card.	Select the SCAN INPUT item from the RECORD menu.

This will show a graphical representation of the input signal.	While

monitoring a good satellite pass adjust the receiver level until none of

the signal peaks touch the top or bottom margins of the graph.	Leave a

little safety margin (1/8 inch) just to be safe.

Once the receiver level has been set,  the next step is to record a

satellite pass to disk.  This is done with the VREC program supplied

with the SoundBlaster card.

For the SoundBlaster Pro use the command:

vrec c:cap /a:mic /m:mono /f:low /s:9620 /l:15 /t:840

where:

c:cap - is the name of the file that you want to write the samples.

Remember the sample file will be quite large.  Around 600 Kb

per minute of recording.  In this case the samples will be

written to the file C:\CAP.VOC

/a:mic  - selects the MICROPHONE input.

/m:mono - selects MONO recording.

/f:low  - sets the LOWPASS recording filter.

/s:9620 - sets the recording sample rate to 9620 samples per second.

Note the if you ask for values less that /S:9619 the VREC

program rounds the value down to around 9500 samples per

second.  /S:9620 is the smallest value that actually

selects a sample clock greater than 9600 Hz.

/l:15   - sets the microphone level to full open.  Use the receiver

output level control to adjust the recording level.

(I don't think this is what this does or is necessary

any more.	It is left over from early attempts to get

rid of hum in the image caused by a bad cable)

/t:840  - says to record for 840 seconds (14 minutes).  This value

can be adjusted to whatever length is desired.

After a satellite pass has been recorded and written to disk as a .VOC or .WAV

sound sample file,  the next step is to demodulate the sound sample file

into an image file.  Use one of these commands:

sbdsp c:cap.voc c:cap.img	    (SoundBlaster)

sbdsp c:cap.wav c:cap.img	    (ProAudio)

This will read the recorded sample file C:CAP.VOC or C:CAP.WAV and write the

resulting image to the file C:CAP.IMG  (remember that these files will

be about 8 Megabytes EACH).  If you do not specify the output file name,

the demodulation program will ask if it is OK to write the output image

over the data in the .VOC file.  This is a VERY time consuming process,

but can be useful if you are short on disk space.  Note that if you

choose to overwrite the input data file,  you cannot experiment with the

various timing adjustments or interrupt the demodulation program.

Once the input file has been demodulated you can use your favorite

image processing program (such as IMDISP, APTCAP, or SATVIEW) to view

the resulting image.  The file written by the demodulation program is

raw binary 8-bit gray scale image.  Each line is 4800 pixels wide.  A 14

minute pass has 1680 scan lines. (Because of the timing adjustments, etc

the output image may be short a few lines).

---

Tweaking in the timing:

What you should see is the APT image complete with the normal "doppler

bow" curve.  The curve in the image is due the speed of light.  Since

light travels at 186,000 miles per second, each pixel of image shift in

the doppler bow (1/9600 of a second) represents about 19.4 miles of

satellite movement relative to the receiver.

As the satellite approaches the receiver the radio signal has to travel

less and less distance to reach the receiver.  As the satellite moves

away the radio signal takes longer and longer to reach the receiver.

The point in the doppler bow where the curve changes slope

is the closest point of approach of the satellite pass.

In a properly adjusted image,  the shape of the doppler bow curve

should appear symmetrical about the closest point of satellite approach.

If the image is slanted more towards the left or right, or appears like

a TV image that has lost sync then you will need to adjust one or

more of the timing adjustment factors.

Usually only the fine timing adjustment (/F) needs to be set.  To get an

idea of the magnitude of the adjustment needed, count or estimate the

number of pixels that the image slants.  (Hint:  use the pan and zoom

features of your image display program.  If your display program

re-sizes the image to fit the screen, remember to count pixels in the

FILE, not those on the screen).  Divide the total file size by the

number of pixels that the image slants from top to bottom.

adjustment factor = (file size / pixels of image slant)

The more the severe image slant is, the SMALLER the timing adjustment

factor value.  Subtle image slants are typically corrected with large

valued adjustment factors (say 50,000 - 500,000).  Adjustment factors in

the thousands would be used for images that slant a good portion of the

width of the screen.  Even more severe slants first should be corrected

by tweaking the /C (coarse) timing adjustment.

Images that slant to the left use a positive adjustment count.

Images that slant to the right use a negative adjustment count.

->|      |<- estimate the number of pixels of image slant

|      |

|       ----------	    ----------

|     /	       /	   \	       \

|   /  image   /		     \	 image	 \

| /	   /		       \	   \

| ----------			 -----------

Use /F+value			Use /F-value

The loss of pixels between DMA sample blocks shows up as small

periodic jags in the image of a pixel or so every 6-8 scan lines.

Experiment with the /O and /E command line options may be to minimize

these remaining jaggies.  For these options,  larger values produce

greater effect.  The maximum DMA adjustment factor possible is +/- 255.

Only + values should be needed here.  The software supports - values

just in case...

The image may look OK except for one or more large and sudden shifts or

discontinuities.  These shifts usually occur on systems with slower hard

drives or CPU's.  The cause is usually the excessive time it takes DOS

to extend or otherwise allocate disk space as it writes the .VOC/.WAV file.

If this occurs in your images, it can usually be fixed by recording the

sample file to a freshly "defragmented" disk.  Use a disk optimizer

program like Norton Utilities or PC Tools to pack the sample file hard

disk drive before running the VREC or RECFILE recording program.

Once the proper timing adjustment factors have been determined for a

particular computer, they should not need to be changed (except possibly

for a drift in the SoundBlaster sample clock rate with temperature).

A batch file can be created that will automatically use the appropriate

timing factors.

--------

Updates for using SBDSP with the MediaVision ProAudio Spectrum boards:

Version 1.1 of SBDSP now supports demodulation of 8-bit audio .WAV files

such as those produced by the MediaVision ProAudio Spectrum and PAS-16 boards.

To use the MediaVision boards you should connect the receiver to the

left channel of the LINE input jack.  The MICrophone input seems to

compand or otherwise mess up the audio samples (just backwards from what

the SoundBlaster seems to do).

Mediavision does not seem to supply a recording level monitor program

like VEDIT2 that can be used to adjust the audio input level graphically.

Experimentation is the order of the day here.  On my system I turn down

all the MIXER inputs from the "PAS *" command except for the EXTERNAL

input jack.  I turn this one up ALL the way and connect a receiver that

produces a 1.0 Volt peak-peak audio output.  This seems to produce

demodulated files that have values ranging from 0..160 or so.  A little

more input level would be nice but my all my receivers have their outputs

fixed at the 1.0 V level required by my A&M (R.I.P) demodulator board.

If your images are excessively dark crank up the input volume.	If they are

too light or distorted or have noise bars,  crank down the input level.  The

line input of the PAS boards is designed for 2.6 V signals.

To record a pass for the MediaVision ProAudio Spectrum 16 board use

the command:

recfile c:cap.wav r9658 8

where:

c:cap.wav - is the name of the file that you want to write the samples to.

Remember the sample file will be quite large.  Around 600 Kb

per minute of recording.  In this case the samples will be

written to the file C:\CAP.WAV	You must manually stop the

recording because RECFILE does not have a record timer feature.

r9658 - sets the recording sample rate to 9658 samples per second.

Note the if you ask for values less that R9658 the RECFILE

program rounds the value down to around 9500 samples per

second.  R9658 is the smallest value that actually seems to

select a sample clock greater than 9600 Hz.

8 - forces 8 bit recording (supposedly the default but let's be a bit

paranoid...)

Everything is then the same as with the SoundBlaster except there should be

no need to use /E or /O DMA block adjustment factors.  The MediaVision boards

do not seem to lose samples between DMA blocks.  Also the .WAV file format

does not have any internal markers that indicate where DMA blocks begin and

end.

-------

There are a couple of new command line options in version 1.1:

/I - produce an output files with inverted pixel values (i.e. swap black

and white).  Very useful with the Infrared (night) imager on the

Russian Meteor 3 series satellites.

/S=scale - Use to adjust the output pixels for the full dynamic range

of 0..255.   If the pixels in the demodulated images only

cover a portion of the desired range of output values,  then

this value can be increased.  The default is 255.

For example if the output images only have pixels with

values from 0 to 128,  a value of /S=512 could be used to

boost the results to 0..255, etc.

Finally SBDSP 1.1 also supports raw binary input files.  These raw binary

files should not have any embedded control information or headers.  A header

at the beginning should not cause any major problems and would be demodulated

as data.  Raw binary files MUST have a .BIN extension.	Normally the files

should contain samples with values centered about 128.	To demodulate files

that have normal signed eight bit values centered about 0 use the (untested)

'/Z' option and hope for the best.

Dallas Remote Imaging Group BBS ====> 214-394-7438

RECEIVING WEATHER SATELLITE IMAGERY: A BEGINNER'S PRIMER

--------------------------------------------------------

Courtesy Dallas Remote Imaging Group

Datalink RBBS ============> 214-394-7438

Dedicated to satellite tracking, decoding of NOAA/Soviet

meteorological satellite telemetry, and Digital Image

Processing of satellite pictures.

Jeff Wallach,      N5ITU, Chairman

John Williams,            Co-Chairman

John DuBois,       W1HDX, VAS/HRPT Design Engineer

T S  Kelso,               Air Force/NASA liaison

Ed O'Grady,               Soviet Space Program Analyst

Mark Sims,                Director Software Development

All aspects of Amateur Radio covered on Datalink RBBS

(IF YOU COPY THIS BULLETIN FOR OTHER SYSTEMS, PLEASE BE KIND ENOUGH

TO LEAVE  THIS BANNER  AS DUE CREDIT TO THE HARD-WORKING FOLKS WHO

DEVELOPED AND CONTINUALLY UPDATE THIS INFORMATIVE 'HOW-TO'BULLETIN)

Dayton HamVention  '91

(rev 14)

Tom  Gentry, K5VOU,  has  been  kind  enough  to  help  us prepare the

following:

HOW TO RECEIVE APT PICTURES FROM THE NOAA SATELLITES

----------------------------------------------------

So  you  have  decided you want to receive the NOAA and Russian METEOR

orbiters  and  you  have  a  radio  that  receives  137.500  MHz and a

computer.  So  now what do you do? I hope this short dissertation will

steer you in the correct direction.

To  display a picture from the orbiting birds you need several things.

First  you  need  to  'acquire' the satellite. This means you need the

program to calculate its position at any time and determine if you can

hear  the  signals from the 'bird'. The best place to get this program

is  probably AMSAT-NA an organization dedicated to AMateur SATellites.

They have several very good tracking programs for the IBM and also for

Commodore  and  Apple as well. After you have acquired the program and

have it running on your computer, you then need the Keplerian Elements

for  the  NOAA  satellites.  These  elements  as well as some tracking

programs  are  available from the CELESTIAL RCP/M, run by TS Kelso, at

513-427-0674 (Fairborn,OH) and from the Weather Fax and Remote Imaging

DATALINK BBS run by Dr. Jeff Wallach, N5ITU as bulletins for some 100+

sets of satellite  data.   Another bulletin also tells you to what the

printed  numbers  correspond  so  you can enter them into the computer

program.  Use  the  newest  elements  you  can  find  as  the orbiting

satellites are subject to forces which change their orbits. If you use

Keplerian Elements more than 3 or 4 weeks old, your chances of hearing

the bird are reduced.

Once you are tracking the birds you should hear them on your receiver.

They  will  be on 137.62(NOAA-9/11), 137.500(NOAA-10), and on 137.850,

137.400, and 137.300 for Soviet Meteorological Satellites/METEOR.  The

Chinese   satellite   Feng   Yun  transmits  on  137.795  when  it  is

operational.  Now comes the first thing different about NOAA orbiters.

The  frequency deviation of the FM transmission is about +/-18-20 kHz.

This  is  4  times  the  normal deviation of a police call and 1/3 the

deviation  of  the  FM  broadcast  band  FM signal. If you receive the

signal  on  the  regular communications width of the scanner the light

areas of the picture will be noisy and the signal will sound distorted

and the picture will basically be useless. If you use the FM broadcast

bandwidth  (assuming  your  scanner  will let you), the signal will be

weak and there may be too much background noise to see the picture.

So  what  now ?   Well  there  are two ways to fix the problem, find a

receiver  with  the  proper I.F. bandwidth filter of 50 kHz, or modify

the  I.F.  of  your scanner to about 50-80 kHz. The simplest mod which

has  been  found to be workable is to remove the narrow communications

I.F.  filter  and  replace it with a 0.01 uFd capacitor. This provides

for  good  Wefax  pictures  from  GOES  and  pretty  fair NOAA orbiter

pictures  -  see  March  1991,  '73 Amateur Radio Today' article for a

description of this procedure. It can of course result in retuning the

radio,  voiding the warranty and making the squelch not work properly.

The  other  characteristic of the NOAA satellite transmissions is THEY

ARE  WEAK.  To  get  good  pictures  over a large area we have found a

pre-amplifier  to  be  essential. A GaAs-FET type can be obtained from

various  sources at a reasonable price but still about 50-100 dollars.

You  can  also build one for about 25 dollars from plans for a pre-amp

for the 2-meter amateur band and tune it to 137.5 easily.

Lastly,  but  importantly,  the antenna can be of an omni directional,

uniform pattern type such as a turnstile antenna similar to those used

in  the  FM  broadcast reception business but of course tuned to 137.5

MHz. Mount the pre-amp at the antenna if possible. Crooked coathangers

on  broomsticks  have  been  known to work, and it is not difficult to

build the antenna. Of course 2-meter beams with Az/El tracking will do

the job well.

Now that you have a nice audio signal from the satellite, what happens

next? The audio tone of 2400 HZ which is the carrier tone that is used

to  carry  the  picture  information (video), must be detected and the

video  data converted from analog to digital and then displayed on the

computer monitor by the software. The hardware and software to do this

is  available  from  several  sources  with more coming along. Several

stand-alone  boxes  are  also  available  that  produce  some  form of

computer  output either in printed form or on the display. In addition

the  people  on the DATALINK BBS may also have some data to share with

you regarding this equipment.

One  of  the  best  sources  for  information is the WEATHER SATELLITE

HANDBOOK  by  Dr. Ralph Taggart. The 4th edition is now available from

the ARRL. A must is THE JOURNAL OF THE ENVIRONMENTAL SALELLITE AMATEUR

USERS'  GROUP - JESAUG - which is published quarterly. It is available

for  $30.00  per  year  from  the  editor, Jeff Wallach, PO Box 117088

Carrollton,  TX.  75011-7088.  Back  issues  of  the  JESAUG  are also

available.

One  good  place  to start is with the Elmer Schwittek MULTIFAX or the

MULTIFAX  MAP  software  which  runs on  the IBM PC.  The PC interface

hardware  is  available  as printed circuit board alone, board and all

parts,  or completely assembled.  Additional image processing software

is available online DATALINK.

The  units  available from David Schwittek, Ralph Taggart, Jerry Dahl,

GTI Electronics and Quorum communications listed under Sources are all

well worth investigating if you are just getting started.

You  can  obtain  directly  from  DATALINK  the MULTIFAX MAP interface

hardware designed by Jim Bartlett and David Schwittek.

'Typical' APT Receiving Station

-------------------------------

[1]   ANTENNA

-

[2]   PRE-AMP

-

-

-

-                          [ STAND ALONE UNITS ]

-

[3]   RECEIVER  ----------------->  FAX MACHINE  ---> PAPER

-                          WRASSE UNIT  ---> MONITOR

-                          YU3UMV UNIT  ---> MONITOR

- - - - VCR

-

-

-

[4] - VIDEO DETECTOR   -

-                  -

-                  -

[5] - ANALOG/DIGITAL   -

-                  -

-                  -            [ SCAN CONVERTERS ]

-                  -

[6] - INTERFACE TO PC  - <=======>  ROBOT 1200

-                  -            TAGGART'S 1700

-                  -            IMAGEWISE

-                  -

[7] -    SOFTWARE      -            ** SECOND DISPLAY **

-                  -               for weather images

-                  -

-                  -

[8] - GRAPHICS ADAPTOR -

-

-

-

-

[9]   PC DISPLAY / MONITOR

Display weather images

(or commands for SCAN CONVERTERS)

-

-

[10]   PC PRINTER / DOT MATRIX / LASER

NOTES:

------

[4] VIDEO DETECTOR, [5] ANALOG/DIGITAL CONVERTER, and

[6] INTERFACE TO PC may be in separate hardware units

or all on one circuit board.

Sources:

----------------------------------------------------------------------

NOAA Data / Keplerian Elements:

DATALINK RBBS, N5ITU, Dr. Jeff Wallach, 214-394-7438,

DALLAS REMOTE IMAGING GROUP ELECTRONIC BULLETIN BOARD

UPDATES ON NOAA BULLETINS, SATELLITE LAUNCHES, FREQUENCIES

DISPLAY PICTURES FROM NOAA SATELLITES ON YOUR OWN P.C.

CELESTIAL RCP/M, TS Kelso, 513-427-0674, NASA Prediction Bulletins

are online and updated several times weekly

Canadian Space Society BBS, 416-458-5907,  2-Line Elements  maintained    |

by Ted Molczan of 600+ satellite - Visual satellite tracking info        |

Satellite Tracking Software:

AMSAT-NA,  PO  Box  27, Washington, DC 20044, Software Exchange,

phone 301-589-6062 (voice), N4HY - QUIKTRAK, W0SL - ORBITS II,III,IV,

and InstantTrack by Franklin Antonio.

Roy D. Welch, W0SL, 908 Dutch Mill Drive, Manchester, MO 63011,

phone 314-391-1127, ORBITS II-CGA, ORBITS III-EGA, ORBITS IV-VGA

DATALINK BBS, download PASSCHED.ZIP, SEESATxx.ARC, TRKSAT.xxx and others

CELESTIAL RCP/M, TS Kelso, 513-427-0674, PASSCHED.ZIP or SEESATxx.ARC

Astronomical Data Service, Rodger Mansfield, 3922 Leisure Lane,

Colorado Springs, CO 80917-SPACE BIRDS program and Newsletter, $40.

Paul E. Traufler, 111 Emerald Drive, Harvest,AL 35749-TRAKSAT Satellite   |

Tracking program-Non - Registered $10.00, Registered Version $25.00.     |

Bill Bard, 1732 74th Circle NE, St Petersburg, FL 33702, MACSAT           |

tracking program for the Apple Macintosh - $10.00 for disk and manual.   |

RPV Astronomy BBS, Dave Ransom, 213-541-7299, STSORBIT Tracking program   |

and latest version of TRAKSAT and NASA Prediction Bulletins online.      |

Receivers:

Vanguard Labs, 196-23 Jamaica, Hollis, NY 11423,

phone 718-468-2720, BBS Dataline 718-740-3911

Hamtroincs, Inc.,65-D Moul Road, Hilton, NY 14468-9535

phone 716-392-9430

DARTCOMM, Mr. N. Hearn, DARTCOMM Ferndale, Postbridge, Yelverton,

Devon PL20 6SY, Great Britain, UK, phone 011 44 0822 88253

Quorum Communications, 1020 S. Main St. Suite A, Grapevine, TX 76051,

phone 817-488-4861, BBS Dataline 817-421-0228                            |

Scanner Modification, See March, 1991 '73 Amateur Radio' Magazine for     |

'Weather Satellite Reception' article by John E. Hoot, page 12.          |

Crystals:

JAN  Crystals,  2400  Crystal  Dr,  Fort  Myers,  FL 33906-6017,

phone 1-800-237-3063 or 813-936-2397

Pre-Amps:

Quorum Communications, 1020 S. Main St. Suite A, Grapevine, TX 76051,

Spectrum   International,Inc.,  Box  1084,  Concord  MA  01742,

phone 508-263-2145

Vanguard Labs, Hamtronics and GLB for Helical RF Filters for intermod

Ehrler-DuBois Very High Performance LNA for 1500-1750 Mhz, contact

Greg Ehrler or John DuBois via DATALINK

Antennas:

ARRL Handbook, 1986, turnstile over ground antenna.

ARRL Satellite Experimenter's Handbook by Martin Davidoff, K2UBC

Modern Electronics, September 1988, Hank Brandli article, page 82,

Receiving Satellite Weather Photos

Hardware Interface to PC:

A&A  Engineering,  2521  W. La Palma, Unit K, Anaheim, CA 92801,

phone 714-952-2114 - both AM dectector and A/D converter, input to PC

via game port (4-bits) or parallel I/O Interface (8-bits) - kit is

$50. - assembled and tested unit is $70.

Microcomm,  H. Paul Shuch, N6TX, 14908 Sandy Lane, San Jose, CA 95124,

phone 408-377-6137 - FAXBOARD - A Weather Facsimile Display Board for

the  IBM  PC  -  Complete  documentation  to build your own interface

circuit  on  an  IBM  PC  prototyping  board - video and an 8-bit A/D

interface - $6.00, also in QEX, Sep '88.

Video Detectors:

RTM  Circuit  Boards,  205  Elm  St.,Van  Horne, IA  52346-0400,

video detector  (the 'Wilson Board')  see QST Magazine, Jan '86 and

Vince Coppola's  article in Oct 1988  73 Magazine

QST Magazine, August 1985, Grant Zehr article, page 27, video detector

circuit

A/D Converter & Parallel I/O Interfaces for the IBM PC:

MetraByte,  440  Myles Standish Blvd., Taunton, MA 02780, Model PIO-12

board for the IBM PC, phone 617-880-3000

John  Bell  Engineering, Inc, 400 Oxford Way, Beimont, CA 94002,

phone 415-592-8411

Sources:

----------------------------------------------------------------------

Display Software and Hardware - 'Amateur':

Multifax,  David  Schwittek,  1659 Waterford Road, Walworth, NY 14568,

phone  315-986-2719, Multifax 5.x and 4.x for VGA, 3.x for EGA on the

IBM-PC. This is the original 16 color system, see Multifax MFMAP.

Clay Abrams Software, 1758 Comstock Lane, San Jose, CA 95124, software

for the ROBOT 1200 - see QST Magazine, Jan '86

Ralph  Taggart,  602  S. Jefferson, Mason, MI 48854, Weather Satellite

Handbook  Scan  Converter - New Model 1700 APT Scan Converter for the

the  IBM PC. Order Handbook from the ARRL, Scan Converter from Metsat

Products,  1257  Glen  Meadows  Ln,  East  Lansing  MI,  48823, phone

517-332-7665.   Also   see   73   Magazine,  Nov  '84  and  Dec  '84,

WSH/Handbook,  and  WEATHERSAT  columns  in 73 Magazine from 1987 and

1988.  Weather Satellite Handbook BBS, 517-676-0368, Ralph Taggart.      |

ImageWise  Display/Receiver,  Circuit  Cellar Inc.  -  Micromint Inc.,

4 Park Street,  Vernon, CT 06066, phone 203-875-2751 - BYTE Magazine,

May,  Jun,  Jul,  Aug 1987. JESAUG 88-2 and Oct 1988 73 Magazine have

information  on  how  to  build  the video and A/D interface. Ver 2.0

software  supports  VGA  graphics and 640 samples per line at 64 gray

levels.  With  VGA  the  ImageWise  Display unit is not required. The

software  is  available for $29.95 from Vince Coppola, N1VC, 6 Bobbin

Road, Terryville, CT 06786

ASAT:  An Apple-based Satellite Imaging System by Grant Zehr, WA9TFB -

QEX  Magazine,  March  '88  -  has  complete  information to build an

excellent  unit  using  the  APPLE  with a Redshift Ltd graphics card

giving 256 * 256 pixels at a full 256 levels of gray.

AMIGA Imaging Software - Dallas Remote Imaging Group

PO Box 117088

Carrollton, TX 75011-7088

Voice 214 394 7325

Uses standard Amiga hardware - no special display

Powerful NASA digital enhancement curves built-in

Digital image processing capabilities

Uses A&A Engineering $49 demodulator

Image saved for use by other graphics programs

Full histogram analysis

Full 8 bit data is stored on diskette or hard disk

Unattended operation

Can be setup in 30 minutes

Call for special pricing

Multifax MFMAP, Elmer and David Schwittek, 1659 Waterford Road,           |

Walworth, NY 14568, phone 315-986-2719. MFMAP software for the IBM PC    |

and  the  Jim Bartlett/David Schwittek/DATALINK  interface  card  for    |

APT/GOES/FAX.  This  is  a fully functional 8 -bit / 256 color system    |

with image data captured directly to disk at 4800 or 3600 samples per    |

second.  Additional software is available online DATALINK and Kelso's    |

CELESTIAL BBS - download APTCAPxx.ZIP and IMDISPxx.ZIP.                  |

A Weather-Facsimile Package for the IBM PC  by Jerry Dahl in April and

May  '90  QST describes how to construct a complete FAX system. Notes

at  the  end  of  article  contain  an excellent list of hardware and

software  sources.  The  completed  card can be ordered directly from    |

Jerry  Dahl,  OFS  Software,  6404 Lakerest Court, Raleigh, NC 27612,    |

phone 919-847-4545                                                       |

WeatherFAX-GTI Electronics, 1541 Fritz Valley Rd, Lehighton, PA 18235,    |

has  a  complete  line  of  satellite receiving equipment,               |

George Isleib,  phone 717-386-4032                                       |

Complete Systems - Hardware and Software:

Dallas Remote Imaging Group

P.O. Box 117088

Carrollton, Texas 75011-7088

214 394 7325

Complete Earth-Scan AMIGA turnkey system ready to put on air

YU3UMV  Digital  Storage  and  Scan  Converter  for  Weather Satellite

Images,  described  in  VHF  Communications Magazine, Winter 4/82 and

Spring  1/83  -  Modifications  are described in the Satellite Users'

Group  Journal - JESAUG - available from Jeff Wallach. Back issues of

VHF   Communications  and  the  YU3UMV  printed  circuit  boards  are

available from Fred Sharp

Loren  Johnson,  P.O.  Box  219,  Cleveland,  MN 56017, system for the

IBM-PC  -  $750,  phone  507-931-4849

WEATHERTRAC[tm] IBM-PC Based Satellite Image Acquisition and Animation

System  -  $1885  for  EGA - $3495 for VGA, Fred Bartlett, Softworks,

Inc,  Allentown,  PA, phone 215-395-4441 or George Isleib, Lehighton,

PA, phone 717-386-4032

Quorum Communications, 1020 S. Main St. Suite A, Grapevine, TX 76051,

phone   817-488-4861,   BBS Dataline 817-421-0228,  offers  a  fully

integrated  systems for  the  IBM  PC  including GOES down converter,

GOES/APT receiver,  Wefax  PC  Adapter  and software. A demo disk is

available  on  request or via the Quorum BBS. Quorum also sells the     |

AVHRR/HRPT system designed by John DuBois and Ed Murashie.              |

Spectrum International,Inc.,  Box  1084,  Concord  MA  01742, phone      |

508-263-2145, is the U.S. representative for Timestep Weather Systems   |

SSC/Software Systems Consulting, 150 Avenida Cabrillo, Suite C,          |

San Clemente, CA 92672 - PC HF Facsimile and PC GOES/WEFAX software     |

and complete receiving systems, phone 714-498-5784                      |

Satellite Meteorology:

Satellite  Imagery  Interpretation  for Forecasters  -  a 3 volume set

published  in  1985  for NOAA now available from the National Weather

Association,  4400  Stamp  Road,  Room  404,  Temple Hills, MD 20748,

301-899-3784,  $32  for  members, $45 for non-members ( Membership is

$20. )

Bibliography - Online DATALINK Bulletins:   (214 394 7438 -- BBS)

NASA Satellite Prediction Bulletins, NASA-PB.xxx

How to use NASA 2-Line Keplerian Elements

AMSAT Bulletins and Weekly Notes / Orbital Elements

NOAA APT NOTES

NOAA User Bibliography

Summary of Soviet Meteorological Satellites by Grant Zehr

How to Build a Weather Satellite Groundstation (This Primer)

Wrasse Dedicated System Review

GOES VAS Imaging System for the IBM PC, John DuBois W1HDX

LNA for WEFAX-VAS-HRPT-GPS, John Dubois and Greg Ehrler

GOES I-M Conference Summary

Ralph Taggart's New  APT Scan Converter and Book

Dartcom 137-138 Synthesized Receiver Packaging, John DuBois

Amateur Radio News from Newsline, Bill Pasternack

How to VISUALLY OBSERVE the Space Shuttle, Ted Molczan

AMSAT InstantTrack V1.00 program description, Franklin Antonio

Review DATALINK RBBS bulletins for current information ====> 214 394-7438

DALLAS REMOTE IMAGING GROUP - DRIG  Voice: 214 394-7325

Data: 214 394-7438

P.O. Box 117088

Carrollton, TX 75011-7088

ATTENTION:  Jeff Wallach, Chairman

|

New or Changed Items = |

|