HP LASERJET IIP		REFERENCE SHEET

file: hp2pinfo.doc ...a word for windows ver2.0b file   page 1 of 4

| Display Message | Description |
| --- | --- |
| 20 error | printer has received more data from computer than it can store (no more printer memory available). ALT-CONTINUE to resume printing. You may want to perform a RESET (07) to clear prt's memory. data loss may occur. add'l prt expansion memory may need to be added |
| 21 error | printer has received page data that is too complex to be processed. simplify the page (remove or redistribute page data) & try printing again. while holding down ALT-click CONTINUE to resume printing...possible data loss |
| 22 error | data transfer interface problem between prt & pc...verify the prt & pc sw are set up to communicate correctly...ALT-CONTINUE to resume operation |
| 40 error | data transfer error...can occur if prt was turned off during data trf or if baud rate is not set correctly...make sure prt baud rate = pc baud rate |
| 41 error | temporary error while printing...last printout may be missing text or incomplete...ALT-CONTINUE to resume printing...if error persists prt service is required |
| 50 needs service | fusing assembly malfunction....attempt to clear error by turning the prt off for 15min if error persists call service |
| 51 or 52 error | internal prt malfunction....ALT-CONTINUE to resume...if error persists call service |
| 53 error | prt has identified an error in expansion memory...run self test to verify that the installed memory = the prt identified memory....turn off prt and verify that the expansion memory is installed correctly...if error persists call service |
| 53 error unit 1 or unit 2 | prt has identified an expansion memory error in either memory board 1 or memory board 2. check installation of memory boards. if error persist call service |
| 55 error | internal prt error....ALT-CONTINUE to resume printing...if error persist call service |
| 57 error unit 1 or unit 2 | amount of expansion memory has been identified which exceeds the printer's usable limits. ALT-CONTINUE to resume printing excess memory will be ignored |
| 61-65 needs service | internal printer malfunction...turn prt off then back on if error persists call service |
| 68 error | error detect in user default memory...ALT-CONTINUE to resume printing...all or part of user default setting will be reset to factory default...if error persist call service |
| 68 need service | memory used for storing user default settings require replacement. any user default settings will be reset to factory default ALT-CONTINUE to resume printing error message will remain display as a reminder that printer needs a service call to be placed. |
| 70 or 71 error | installed font cartridge is not compatible with printer...turn off printer and remove cartridge |
| 72 needs service | font cartridge was removed from printer while cartridge was being accessed. turn off printer, reinstall cartridge and turn printer back on. if error persist call service |
| 79 xxx service | internal printer error. turn printer off for 15 min and then back on if error persist call service |

HP LASERJET IIP  	USER'S QUICK REFERENCE

file: hp2pinfo.doc ...a word for windows ver2.0b file    page 2 of 4

| Abbreviation | Description |
| --- | --- |
| A4 | PAPER SIZE 210 MM X 297 MM |
| AUTOCONT | AUTO CONTINUE |
| C | FONT CARTRIDGE |
| C5 | ENVELOPE SIZE (162 MM BY 229 MM) |
| COM10 | ENVELOPE SIZE (4 1/8 " X 9 1/2") |
| DEVICE CONFIG | DEVICE CONFIGURATION (MENU) |
| DL | ENVELOPE SIZE (110MM BY 220MM) |
| DTR | DATA TERMINAL READY |
| EP-L | EP-L CARTRIDGE |
| EXEC | PAPER SIZE 7 1/4" BY 10 1/2" |
| FC | FONT CARTRIDGE |
| FE | FONT ERROR |
| FONTNUM | FONT NUMBER |
| FONTSRC | FONT SOURCE |
| I | INTERNAL FONTS |
| I/O | INTERFACE (INPUT/OUTPUT) |
| JOB SIZE | DEFAULT PAGE IMAGE SIZE |
| L | LANDSCAPE ORIENTATION |
| LC | LOWER CASSETTE |
| LE | LOWER ENVELOPE TRAY |
| LEGAL | PAPER SIZE 8.5" BY 14" |
| LETTER | PAPER SIZE 8.5" BY 11" |
| LINES OF TEXT | LINES OF TEXT |
| MAN | MANUAL FEED |
| ME | MANUAL FEED ENVELOPES |
| MF | MANUAL FEED |
| MONARCH | ENVELOPE SIZE 3 7/8" BY 7 1/2" |
| MP | MULTI-PURPOSE TRAY |
| ORIENT | PAGE ORIENTATION |
| P | PORTRAIT ORIENTATION |
| PARALEL | PARALLEL INTERFACE |
| S | SOFT FONTS |
| SERIAL | SERIAL INTERFACE |
| SYM SET | SYMBOL SET |

HP LASERJET IIP			ATTENDANCE MESSAGE

file: hp2pinfo.doc ...a word for windows ver2.0b file     page 3 of 4

| Display Message | Description |
| --- | --- |
| blank display | internal prt problem...turn prt off for 15min and back on...if problem presist call svce |
| 00 ready | printer is ready to use |
| 02 warm up | prt is warming up....if no change to 00ready mode in 3min call service |
| 04 self test | printer is continuous self test mode...click ON LINE to terminate the testing remaining buffer page will continue to print before printing stops |
| 05 self test | printer is performing the non-printing portion of self test |
| 06 font printout | printer is printing a font listing |
| 06 self test | printer is printing the printout portion of self test |
| 07 reset | prt is resetting erasing any unprinted page data (clear prt buffer) clearing temporaty font & macro & returing to printing menu to the user selected default settings |
| 09 menu reset | same as 07 reset except that the printing menu is returned to factory default setting |
| 10 reset to save | you changed a menu setting with data in print buffer...2 choices exist: 1>save menu changes but lose print data currently in printer ALT-RESET 2> resume printing using previous menu settings click ON LINE. menu change is not saved until printing stops & a reset is performed |
| 12 open or no ep | paper path door is not closed or ep-l cartridge is not installed...close paper path door or install cartridge...if error persist call service |
| 13 paper jam | printer detect paper jam...remove paper jam (paper path door must be opened & closed )click ON LINE to resume printing |
| FC removed | font cartridge was removed while printer was off line & contained print data in buffer...re-insert the SAME cartridge and click ON LINE |
| FE cartridge | font cartridge was removed while printer was on line. the printer must be turned off to clear the error |
| MF feed | printer is configured for manual feed (or received a manual feed command from computer) & has data ready to print. feed the correct paper size into the mp tray & click ON LINE |
| MF ready | printer is ready & configured for manual feed mode |
| MFload | printer is ready to feed paper from mp tray but tray is empty or differetn size is needed. load correct paper into tray. (if mp load continues to flash you need to change mp size menu item to match size requested by the job...click MENU click +/- to display the correct mp size click ENTER) click ON LINE to resume operation |
| LC load | place paper in lower tray & insert tray into the lower cassette |
| LE load | place envelopes into lower envelope tray & insert the tray into lower cassette. a menu will come automatically click +/- to display correct envelope size. click ENTER & then ON LINE |
| MP, LC or LE empty | printer is not using this tray (mp, lc or le) which happens to be empty. NO ACTION IS REQUIRED since the tray is not selected. message is simply a reminder to fill at your convenience. |

HP LASERJET I I P			CONTROL PANEL

file: hp2pinfo.doc ...a word for windows ver2.0b file  page 4 of 4

| * | on line RESET | menu PRINT FONT | enter TEST | message to user window |
| --- | --- | --- | --- | --- |
| ** | form feed CONTINUE | ALT | + - |  |

* = THIS IS ON LINE INDICATOR

** = THIS IS FORM FEED INDICATOR (DATA IN PAGE BUFFER)

ON LINE	places the printer on-line or off-line

FORM FEED	printes any page data remaining in printer (ie:print buffer page data) the form feed
		indicator lights to indicate data is in page buffer.

MENU		enter the printing/configuration menus.

ENTER		selects a new menu-item choice as the user default.  and * appears in the display.

ALT		when pressed in conjunction iwth another key, the lower half of hte other key is preformed.

ALT +/-			steps through the menu item choices.

ALT - PRINT FONTS	prints a list of the available fonts.

ALT-TEST		performs the printer self test, includes printing self test printout.

ALT-RESET		[1] performs a 07 reset return all printing menu items except mp size to their user selected
			defaults and clears print data, temporary softfonts & macros from printer.

[2] HOLD DOWN KEY 5 OR MORE SECONDS performs a 09 reset, return all printing
			menu item except mp size to their factory defaults & clears print data, temporary softfonts &
			macros from printer.

ALT-CONTINUE	clears most error messages & places the printer on-line. allows printer to continue
			printing after stopping on certain types of errors.

TONER  PART #92275A

this toner is also good for hp laserjet seriesIIP, IIP Plus, IIIP

info per printer quick reference card 6/1989