___________________________________________________________________________

STAC FAX                                                           STAX FAX

Title: Stacker and Anti-Virus Programs

STACKER NOTE TEC037 - 6/22/92                          STAC FAX Index #2403

___________________________________________________________________________

Background.

Stacker is compatible with anti-virus programs such as those from Norton

(NAV) and Central Point (CPAV).  However, in order for them to peacefully

coexist, there are a few guidelines to follow.

Installing Stacker.

Before installing Stacker, disable any TSRs and Anti-Virus programs that

are memory resident.  Device drivers such as NAV&.SYS, NAV_.SYS, and

VWATCH.SYS should be disabled by REMarking the drivers in Config.sys

and rebooting.

NOTE: Failure to disable these could result in an incomplete

installation.

Configuring Device Drivers.

It is generally recommended that any anti-virus devices be loaded AFTER

the Stacker device driver in Config.sys.  Loading devices such as

NAV_.SYS before Stacker may cause the system to hang while loading the

Stacker device driver.

Immunization.

CPAV allows you to "immunize" executable files by adding signature

information to the file. Do NOT immunize the Stacker device driver files.

These files use their own signature information for a "CheckSum" self-

check at boot time. Immunizing the files disrupts this checksum

information and prevents them from loading.  Exempt the files from

immunization by adding them to the immunization exception list.  Add the

following files:

STACKER.COM

SSWAP.COM,

STACKER.XT

STACKER.AT

STACKER.MC

If these files have been immunized accidently, restore them from the

Stacker diskette or remove the immunization from the files.

BOOTSAFE from Central Point Anti-Virus.

Bootsafe is Central Point's partition table and boot sector security

system. Usually loaded in Autoexec.bat, it examines the boot sector and

partition information of the default drive (usually C:).  If Stacker is

swapping drive letters, Bootsafe will attempt to read this information

on the Stacker logical drive and report: "WARNING partition/boot sector

has been modified!"   This solution is simply to make Bootsafe look at

the physical boot partition.  The syntax is:     BOOTSAFE d:

(where d: is the letter of the SSwapped boot drive.)  For example, to

determine the physical boot drive, use the Stacker command.  Type

C:\STACKER\STACKER

The Drive Map display will be similar to the following:

STACKER - 2.00, (c) 1990-91, Stac Electronics, Carlsbad, CA

Drive C: was drive D: at boot time  [ D:\STACVOL.DSK = XX.X MB ]

Drive D: was drive C: at boot time

The physical boot drive (D: in this case) is the drive that "was drive

C: at boot time".

Therefore, in Autoexec.bat the Bootsafe line would be:

Bootsafe D:

___________________________________________________________________________

Copyright 1992, Stac Electronics                    STAC FAX (619) 431-6712