���  ���             RELEASE NOTES                     Pre-Release Version 0.5

�ͻɻ����                                                   September 25, 1990

�ͼȼ��ͻ����ͻ�ͻ   OVERVIEW:

�ͼ�����ʼ �   SofScan is a storage media laboratory test utility; one

you might expect to find in use by a certification/refurbishment facility.

SofScan tests DOS formatted media response under user controlled soft error

threshold levels, reporting the slightest discrepancy.  Other disk test soft-

ware that relies on the BIOS for hardware status will usually locate only the

unrecoverable (hard) errors.  The job of the disk hardware and BIOS is simply

\"whatever it takes... get the data\".  Usually the BIOS design does not com-

plain to DOS or concern the user with the expected occasional recoverable

(soft) error.  The hardware and BIOS may only, after exhaustive \"grinding\",

return an error status in lieu of the data. This good/bad method unfortunately

masks the marginal soft error media surface defects. Ignored, many of these

soft spots deteriorate and the user is informed (only after it is too late)

with the old familiar prompt \"Abort, Retry, Fail?\".

SofScan scrutinizes the disk read operation fully, thus flushing out the up

and coming media errors.  Periodic use allows the discriminating PC user to

maintain an above average confidence level in his storage media.  Many

previously unsalvageable hard disks have been placed back in service with the

help of SofScan.

���  ���               FEATURE SUMMARY:

�ͻɻ����              SofScan is a multi-step test process.  A minimum of

�ͼȼ��ͻ����ͻ�ͻ     four scans repeatedly READ the disk test area.  The

�ͼ�����ʼ �     final critical step, marking-BAD clusters, is turned

over to Norton Utilities DT-Disk Test.  As many as 999 scans can be used to

certify continuous duty capable DOS media storage logical units. Any single

error incidence is trapped and reported. Files using error prone clusters are

reported by cluster number and file name. User settings customize the error

sensitivity level.  A continuous dated record of cluster error history may be

maintained on the SofScan diskette.  The error list is written as a batch file

named CLUSTERS.BAT on drive A:.

Norton's DT-Disk Test has an often overlooked command line option (/Cnnn) that

manually marks clusters BAD.  Conversely, GOOD clusters which are marked BAD

may be returned to service with the switch /Cnnn-.  DT verifies that the

to-be-marked cluster is not INUSE by a file.

Upon detection of an INUSE soft cluster, SofScan locates the complete path and

file name to include in the error report.  This feature facilitates the manual

process of using DOS COPY to move each file to safety. The original must be

erased to release the INUSE soft cluster for marking BAD.  Always mark any

UNUSED BAD clusters before moving the associated files of INUSE BAD clusters.

Of course you may mark the affected clusters by any other means available.

���  ���               OPTIONS DESCRIPTION:

�ͻɻ����              The Soft Error Threshold option directs SofScan to

�ͼȼ��ͻ����ͻ�ͻ     lower or raise the error determination level.  Level

�ͼ�����ʼ �     [0] might cause SofScan to issue false error reports.

Alternately, Threshold Level [9] is capable of masking minor defects.  By

definition, soft errors are often inconsistent.  Continued testing is recom-

mended until new errors fail to appear. Using the Beginning and Ending Cluster

options you can limit testing to one area. Passes through the test clusters

may be set from [4] to [999] to facilitate overnight or even week-long testing

sessions.  SofScan's unique process traps and reports any single cluster read

retry.  The estimated test time is updated following any option selection

affecting the total run time.

Test Clusters previously marked BAD [ReTested] will verify marked-BAD

clusters.  Thus allow the user to mark GOOD a cluster if it appears to be

mistakenly marked-BAD. Do not be whimsical about marking-GOOD a BAD cluster;

soft errors are inconsistent.

Caching controllers read the entire track when any cluster on the track is

requested. Therefore, all clusters on a track with a real error will, in

succession, report as BAD.  If the Passes setting is less than the number of

clusters per track, additional sessions will continue to tag the remaining

clusters of the bad track.  Disk Caching Software distorts the cluster

granularity even further.  It must be removed to obtain valid test results.

Error level 32512 is forced for very bad clusters.  This level may also be

reported erroneously if timer interrupts are masked by the Disk BIOS.

���  ���             TESTING OPERATIONS - PHASE ONE:

�ͻɻ����

�ͼȼ��ͻ����ͻ�ͻ   Requires: SofScan.EXE in Diskette Drive A:

�ͼ�����ʼ �

1. Remove all Disk Cache, TSRs, etc.  The easy method is to boot DOS from a

floppy which bypasses the normal CONFIG.SYS and AUTOEXEC.BAT hard disk

startup routines.  You may make a boot floppy entering: FORMAT A:/S

2. Log to the drive to be tested (SofScan only tests the default drive).

Insert the SofScan program and test report diskette in drive A: and enter:

A:SofScan

3. Select test options from the menu and press G to Go or Q to Quit.

4. As SofScan tests the disk the current pass and minutes required are shown.

The current test cluster displays at the right edge of the screen.

5. After the last pass, SofScan will pause to allow you to review the screen

error listing.  A duplicate is saved to the output file CLUSTERS.BAT.

SofScan's command line options are /B,/E,/S,/P,/T for Beginning, Ending,

SoftErrorThreshold, Passes and BadTesting respectively.

���  ���             MARKING OPERATIONS - PHASE TWO:

�ͻɻ����            Requires: Norton Utils DT-Disk Test 4.00 DOS 3.30 dn

�ͼȼ��ͻ����ͻ�ͻ             Norton Utils DT-Disk Test 4.50 DOS 3.31 up

�ͼ�����ʼ �             A:clusters.BAT

1. Ensure that DT-Disk Test is on your hard disk and can be found by the PATH.

You may review A:CLUSTERS by entering: A:clusters    [with no keyword]

2. Mark UNUSED BAD clusters, if any, first by entering: A:clusters MARKBAD

3. If INUSE bad clusters are reported move each file to a safe location using

DOS COPY if possible.  (If not, STOP and seek professional assistance in

recovering critical data.)  Erase each original file to release the INUSE

BAD clusters to be marked BAD.  Enter: A:clusters FILESMOVED

4. If GOOD clusters are found to be marked-BAD and you are confident they

are dependable, enter the following:  A:clusters MARKGOOD

������������������������������������������������������������������������������Ŀ

�Thank you for your support!  If you have found SofScan useful, the licensing  �

�fee of $19 via CreditCard will get you a full function version immediately.   �

�We also distribute UPS COD or check prepaid USMAIL.  Please call:             �

�Allen Swann, 162 Lincoln Ave., Winter Park, Florida 32789 (407) 695-8859      �

��������������������������������������������������������������������������������