*******************************  AILANBIO.DOC  ********************************

PRODUCT:  AILANBIO.EXE V4.05.02

DATE:     June 2, 1993

PURPOSE:  The purpose of the NO_MULTICAST switch is to allow LANtastic

machines separated by a bridge/router (that do not pass multicast

packets but only pass broadcast packets) to communicate.

PROBLEM REPORT NUMBER:  T03590

******************************************************************************

EXPLANATION:

Starting with NOS v5.0, AILANBIO.EXE went to a MULTICAST address format as

oppose to a BROADCAST address format.

For a 6 byte node address a BROADCAST packet would be FFh FFh FFh FFh FFh

FFh.  A MULTICAST packet would have a 1 in the Least Significant Bit of the

first byte, in other words XXXXb XXX1b XXh XXh XXh XXh XXh for ETHERNET.

Since Token-Ring addresses are bit swapped, it would be 1XXXb XXXXb XXh XXh

XXh XXh XXh.  MULTICAST addressing allows a single machine to communicate to

multiple machines ... it provides a "broadcast" type of service without

actually broadcasting to everyone because only the machines with the

MULTICAST address will process the packet.  This provides the distinct

advantage of reducing the processing of "broadcast" queries on a network.

These BROADCAST packets can sometimes play havoc on a LAN or WAN.

Unfortunately, many BRIDGEs and ROUTERs do not pass MULTICAST packets as

"broadcasts", they only pass BROADCAST packets as "broadcasts".  This

prevents machines separated by a BRIDGE or ROUTER from querying or

communicating with each other.

This is a patch version of AILANBIO.EXE (V4.05.02). This version adds

a new switch; NO_MULTICAST. The NO_MULTICAST switch tells AILANBIO.EXE

to send broadcast packets instead of multicast packets.  Aside from the

added switch, there is no difference from the shipping version 4.04 of

AILANBIO.EXE.

Syntax:

AILANBIO /NO_MULTICAST

RECOMMENDED INSTALLATION INSTRUCTIONS:

Copy this AILANBIO.EXE over your current AILANBIO.EXE, so there is no question

which version you are running.

If you have any difficulties, please contact Artisoft Technical Support at

(602) 670-7000.

Brand names are trademarks or registered trademarks of their respective

companies.

LANtastic is a trademark of Artisoft, Inc.  All rights reserved.