SuperPhreek 5.0 Documentation and instructions.

Written by Terrible Terry the Tyrant.

C O N T E N T S

I   Introduction

II   Features

III   The file menu options

IV   The modem menu options

V   The setup menu options

VI   The run menu options

VII   What next?

VIII   Credits

- I - INTRODUCTION

SuperPhreek 5.0 is a utility that uses your modem to locate other

remote modems connected to computers in auto-answer mode. Many

computer systems are connected to the outside world via a modem and

telephone line. SuperPhreek will sequentially scan for those

computer systems. When SuperPhreek connects to a remote system, it

immediately hangs up and logs the telephone number to a user-

definable device, which is usually a disk or printer. If

SuperPhreek does not find the dialed number to be a remote system,

it hangs up and goes on to the next number in the series. Because

some telephone numbers are not assigned to anyone, SuperPhreek

dials from user-assigned blocks of telephone numbers called "sets".

Fifteen sets are available for your dialing pleasure. It is these

sets that allow you to skip blocks of numbers that you do not wish

to call. SuperPhreek incorporates pull-down windows and supports

a mouse. SuperPhreek will run on any PC with 256K and a modem.

- II - FEATURES

SuperPhreek supports the use of comports 1 through 4, and baudrates

up to 9600. All parity, stopbits, and databits configurations are

supported. SuperPhreek supports most any video card, and will

adjust accordingly. A special temp file records every 100th number

dialed, in case of power failure. You may stop SuperPhreek at any

time and quit the program, without loosing any information. All

configuration data is saved to the currently selected filename upon

exit. When you re-run SuperPhreek, you should re-load the desired

configuration file. If you did not select a configuration filename,

SuperPhreek will assign "NONAME.SPC" and "NONAME.DAT" for your

default data files. You may re-load any configuration file you had

previously saved if you like, or let SuperPhreek use its default

NONAME files. SuperPhreek is BNU and X00 compatible.

- III - The file menu options

The file menu options allow you to LOAD and SAVE any special

configuration. If for instance you have SuperPhreek setup with

special sets of numbers, delay rates, Hayes commands etc. you may

wish to save them for use when you return to re-run SuperPhreek

sometime, especially if you would like to start dialing where you

left off. To do this, setup SuperPhreek the way you want it, and

select the SAVE option in the files menu. Select a filename you

like, or let SuperPhreek use its own NONAME.SPC filename. This is

done by pressing the TAB key until the input line is highlighted.

You may now enter a filename. Now, to save all configuration data

for next time, press the TAB key again until the Ok is highlighted.

Now press RETURN. You configuration data will be saved. Now, the

next time you want to resume dialing with the same configuration

data, you may re-load the data you need. Remember, SuperPhreek will

automatically re-load NONAME.SPC when it is first run. If

NONAME.SPC exists, that configuration data will be loaded. You may

still load a different configuration file anytime you like. Be

warned that if you load a new configuration file, your old one will

NOT be saved. You must save it yourself if you want to keep the

current version of it. Generally speaking, the LOAD option works

the same as the SAVE option does as far as functionality. Remember

to use the TAB key to get around in the LOAD and SAVE windows. Be

sure to TAB over to the Ok and press RETURN to save or load.

SuperPhreek supports the SHELL command. To SHELL to DOS, press

RETURN when the SHELL option is green in the file menu. Type EXIT

to return to SuperPhreek. Technical: High was set to 655360 -

150000. Low was set to 0. The stack was left alone also.

- IV - The modem menu options.

SuperPhreek supports com ports 1 through 4. Select the COMPORT

option on the modem menu to select a comport. Use your TAB key to

get around the COMPORT window. The other modem menu selections are

the same. Use combinations of TAB and up/down arrows to get around.

Highlight the Ok and press RETURN to activate you selections. It

is suggested that you use 8 databits, 1 stopbits, No parity and

1200 baud. This is SuperPhreek's default setup. Please see that the

correct comport is selected before you start SuperPhreek to

dialing. SuperPhreek defaults to comport 2. If your modem works on

comport 1, then please change comports. Use SAVE in the files menu

to save that configuration if you like, or SuperPhreek will do it

for you automatically using the NONAME files.

- V - The setup menu options.

There are four options in the SETUP menu. The first one is the Init

Strings option. This option allows you to select Hayes modem

strings to suit your modem. Please refer to your modem manual for

further information on Hayes modem commands. Basically, it is these

Hayes modem commands that tell your modem how to act. Not all

modems are created equal, and some may need a configuration

different than SuperPhreek's default Hayes strings modem

configuration. The Init Strings describes the strings' jobs. The

first is issued when SuperPhreek first runs. This is a good place

for the ATM0 or ATM1 command. This will turn off or on you modems

speaker respectively. The second field is the string to send when

dialing each time. This is where your ATDT or ATDP command will go.

These commands dial TONE or PULSE respectively. Tone is the

default, although some people do not have tone dialing lines, and

they can change this field if that is the case. The third and final

field is the Hayes command to be send on exit of SuperPhreek. This

is usually something like ATZ. It might be something to reset the

modem to the way it was before SuperPhreek woke up. All three of

the fields here in the Init Strings window allow 127 characters,

although not many modems will allow such a long command. Please use

a combination of the TAB and arrow keys to navigate the window. To

get to the #2 strings line, TAB over to it. You will know you are

on it when it gets bright. You can now edit it, by pressing the

arrow key to the right or you can just start typing and re-do

everything in the line. TAB over to the Ok symbol and press RETURN

to activate your selections. The second option on the setup menu

is the Redial Time option. This is the time that SuperPhreek allows

before it redials the next number in the series. As with all other

options described above, you will need to use a combination of your

TAB and arrow keys to navigate this window. Select the appropriate

speed and TAB over to the Ok and hit RETURN to activate.

SuperPhreek's default setting here is the best by far. Unless you

have a very good reason, leave this option alone. The third option

on the Setup menu is the Numbers option. This is where you will

setup your dialing sets. These are numbers that SuperPhreek will

dial from to look for a remote system. There are fifteen sets here.

SuperPhreek will first look to set #1 to dial from. If there are

no entrees in set #1, it will go to set #2 and so on, until it

finds a valid set. There are three fields for each of the fifteen

sets in the Numbers window. The first field is any long distance

prefix that you might want to dial before the actual 7 digit

phonenumber. You of coarse do not have to have a long distance

number in this field, and actually may never need to, unless maybe

if it is a 1800 long distance number, which would make it a free

call anyway. This first long distance entry field will never be

incremented, as it is only added to the beginning of the actual 7

digit phonenumber for dialing. If you are dialing local

phonenumbers, you will not have anything in this first field at

all. The second field in the Numbers window is the 7 digit

telephone number to start dialing from. This number will be the

first number that is dialed. It will be dialed, and incremented

until it reaches what is in the third field in the Numbers window.

Once a set has expired (the START phonenumber is equal to the STOP

phonenumber), that set will never be dialed again, and the next set

will be called upon. Remember, the START phonenumber is the second

field, and the STOP phonenumber is the third field. To modify any

of these fields, simply use you TAB key to tab over to the set and

field in that set you would like to modify. The long distance field

(the first field in any given set) allows 127 characters to be

entered, which is way more than enough. The second and third fields

(the START and STOP phonenumbers) in any given set allows a 7

character number. To make the changes permanent in the Numbers

window, TAB over to the Ok message at the bottom of the screen, and

hit RETURN. Type an ESCAPE to escape with no changes. Once you have

a setup you like, you might consider Saving it from the Files menu.

If you do not, SuperPhreek will save it for you, and automatically

re-load the setup for you. If you exit the program in the middle

of a dialing session, SuperPhreek will know where to re-start

dialing from. The third option in the Setup menu is the Output Data

option. This window allows you to configure several very important

data collection options. There are two clusters of options at the

top of this screen, and two input lines down below. Cluster #1

allows you to set the output device for where the hits (connections

to remote systems) have been made. Legal options here are the

printer, file, or screen. By default, the screen is the only device

active. You will use a combination of your TAB and arrow keys to

navigate this cluster. Use your space bar to activate and

deactivate options in this first cluster. If file is selected, all

hits will be logged to disk. The filename of the file to log hits

to is in the second input line at the bottom of this Output Data

screen. Use you TAB key as usual to navigate to it for

modification. SuperPhreek's default filename to save the hits to

is NONAME.DAT. You do not have to change this filename if you do

not want to. If this file exists, it will not be overwritten, only

appended to. If this file does not exist, it will be created for

logging hits to. If you toggle the printer option in the first

cluster in the Output Data window, the printer will also be used

to log hits. Any device that has a X by it in this first cluster

will be used for logging. The printer device may vary from user to

user, so the second cluster option was added to SuperPhreek. This

second cluster allows you to select a printer device. Valid devices

are PRN (the most common and the default) or the AUX or the PRN2

devices. If you decide to play around and select a device that is

non-existent, you will receive a big fat system crash. SuperPhreek

does allot of checking (called idiot-proofing), but it will not

kiss your ass! Use some responsibility. If you don't know what your

printer device is, then it is most likely the default setting of

PRN. Leave this option alone if you don't know anything about what

I am talking about. So, basically, the first cluster (output hits

device(s)) works with the second input line in the Output Data

window. Cluster #2 in the Output Data window is only looked at if

the printer device is to be logged. If you do not have the hits

logged to the printer, then this second cluster (printer device)

will never be looked at. You MAY have SuperPhreek log hits to one

or more of the options in the first cluster (the output hit to

option). For instance, you can have SuperPhreek log all hits to the

screen, the file named in the second input line, and the printer

device named in the second cluster box. Once you have SuperPhreek

setup the way you like it, and are ready to commence scanning for

remote systems, you might consider saving all the selected options.

If you do not, SuperPhreek will save it for you, and re-load it

upon re-execution. The HELP option in SuperPhreek is for quick

overviews of what is what. Several Function keys are active. See

the Help window selection QuickKeys.

The data that is output when a system is encountered is simple.

First the phonenumber, then the set that was dialed from, then the

time and date of connect, then the number of hit is was - if it was

the 10th hit, there will be a 10 here.

- VII - What next?

Once you have a nice collection of telephone numbers in your file,

or on your printer paper, SuperPhreek's job is done. What you do

with this information is not my business. You may dial these

computers up if you like, and see what they are all about. You

might let DataBase Hacker do it for you though. DataBase Hacker was

written by another local, Lord Random. I understand that the next

version of DataBase Hacker will not only dial a remote system up

and interrogate it, but give you passwords when it is done for your

exploring pleasure. I also understand that the next version of

DataBase Hacker might have an option to allow the use of

SuperPhreek data files to be uses as a source of dialage, meaning

that DataBase hacker will use the list of telephone numbers output

by SuperPhreek to dial from, and interrogate those systems.

- VIII - Credits

Lord Random and The Master Cylinder are as responsible for giving

me the idea to write SuperPhreek 5.0. Had it not been for several

arguments we have had about programing languages, I might never

have wanted to prove that Pascal is by far the easiest language to

program in :) :). The async routines are the popular ASYNC12

routines written by Mark. The program was written in Turbo Pascal

6.0 using only object oriented programing routines from the Turbo

Vision library. Other than these two libraries used, the bulk 100K

of SuperPhreek is code by me, Terrible Terry the Tyrant.

SuperPhreek contains some 4000 lines of original code. If you were

to include all of the units and libraries used, the code would

surly exceed 20,000 lines. Please note that I did not have to

borrow from Turbo Vision, or Async12, but did, and SuperPhreek is

all the better for it. The source code is unavailable - sorry, you

try to write one!

There will not be a SuperPhreek 5.x or 6.0, as this version is

about as good as it gets.

Terrible Terry the Tyrant - 1/92