# Plug and Play Device Driver Interface

# for

# Microsoftâ WindowsÔ 3.1 and MS-DOS

Version 1.0
June 2, 1993

Plug and Play Device Driver Interface Specification

for

Microsoftâ WindowsÔ 3.1 and MS-DOS
ã Copyright 1993 by Microsoft Corporation.

µ

Introduction	4

Usage model	4

Configuration-management utilities	5

Installation programs	6

Device drivers	6

Data Structures	6

Device ID	7

Bus access handle	7

Configuration buffer	8

Resource attribute fields	8

Memory Range	8

IRQ	9

DMA	9

API	10

Calling Conventions	10

16-Bit Calling Convention	10

VxD Calling Convention	10

Device Driver Interface (DDI)	11

Presence Detection	11

16-Bit Code	11

Virtual Device Drivers	11

CM_GetVersion	12

16-Bit Code	12

Virtual Device Drivers	12

CM_GetConfig	13

16-Bit Code	13

Virtual Device Drivers	13

CM_LockConfig	14

16-Bit Code	14

Virtual Device Drivers	14

CM_UnlockConfig	15

16-Bit Code	15

Virtual Device Drivers	15

Example	16

# Introduction

The following document describes the interface which will allow hardware and software vendors to develop  installation programs, configuration utilities and device drivers for Plug and Play (PNP) devices. By following this specification vendors can ensure upward compatibility with future versions of Windows. We have made a conscious effort to simplify the PNP interface to minimize the impact on existing device drivers and to allow smooth upgrade when PNP gets tightly integrated with the operating system.

The specified API is bus independent, any bus-specific functionality should be handled via existing mechanisms. For example drivers should access the EISA configuration space by issuing an INT 15 call, and access to PCI configuration space should be done using INT 1A. To maintain bus-independence we will support configuration management of only the four standard system resources: I/O, IRQs, DMA, and memory ranges. Any other configuration management is beyond the scope of this specification.

# Usage model

The APIs described below can be used by, or provided to the following clients:

·	Configuration management utilities: can provide configuration services to installation programs and device drivers. These utilities should be implemented as a real-mode/protect-mode VxD pair. Both drivers will reside in the same binary. This will ensure the same calling convention will work for DOS applications and Windows VxDs. These utilities will typically go through an installation phase during which they will determine which resources  are statically used by "old" devices. At the end of the installation process, these utilities will offer the services outlined below. Once a certain allocation has been handed out to a client via the CM_GetConfig API, the configuration utility is responsible for setting the relevant device to the assigned configuration.

·	Installation programs: when a new device is installed whether it's PNP or not, an installation program can ask a configuration management utility to enumerate currently installed devices, and request a valid resource allocation given all possible combinations supported by the new device. In the case of PNP devices, it will enable automatic configuration without any user intervention.

·	Device drivers: by issuing the CM_GetConfig call, device drivers can support dynamically assigned configurations. This will ensure maximum flexibility when other devices are added to the system at a later stage. This is also the method by which resources will be allocated to device drivers in future versions of Windows. In order to access bus-specific or vendor specific information, device drivers should continue using existing mechanisms such as INT 15 for EISA, and INT 1A for PCI. To access ISA PNP devices, drivers will need to locate their device and issue the commands defined in the Plug and Play ISA Specification 1.0.

The following figure describes a possible configuration framework that uses the proposed interface:

µ §

## Configuration-management utilities

A configuration management utility must provide all the services described in the API chapter. To ensure consistency across different implementations and to be upward compatible, configuration management utilities should adhere to the following naming and VxD ID guidelines:

VxD ID:	0x34

Common name:	OLDCFGMG.SYS

During its setup a configuration management utility will typically perform the following operations:

Þ	Detect currently used resources and store in a configuration file or non-volatile storage. The detection process may involve looking at .ini, .bat or .sys files, reading or writing to I/O ports and accessing bus-specific configuration spaces.

Þ	Copy configuration management binaries, and make sure the real-mode portion will get loaded before any other DOS device driver

Þ	Reboot the system

During and after boot a configuration management utility will typically go through the following sequence:

Þ	Access a configuration file or non-volatile storage and initialize data structures

Þ	Utilize bus specific configuration schemes to assign previously allocated resources to devices under its control. If there is BIOS support for these schemes, the configuration utility will typically utilize these BIOS services

Þ	Respond to CM_GetConfig calls with previously assigned resource allocation

Þ	Respond to CM_GetVersion with appropriate information

Þ	When issued the CM_LockConfig verify that the requested resources are available and update configuration file or non-volatile storage. Responding positively to this call locks the specified configuration for the related device

## Installation programs

Installation programs will need to be modified to take advantage of the new configuration services. These services enable installation programs to get an accurate count of the currently installed PNP devices and to receive a conflict free allocation given a set of constraints. These programs will typically go through the following sequence:

Þ	Issue the CM_GetVersion call to verify existance of a PNP configuration management utility

Þ	Issue the CM_GetConfig to verify the PNP device has been correctly installed and detected. This call is optional, installation programs can jump to the next step directly

Þ	Lock a certain configuration via the CM_LockConfig (assumes pre-known device ID or use results of CM_GetConfig call). Note that CM_LockConfig is not required for ISA PNP devices. By the time the installation program has launched, the configuration utility has already read the device's configuration space and assigned a valid configuration to the device.

Þ	Copy binaries and initialization files

## Device drivers

Device drivers will need to change their initialization sequence to dynamically determine their configuration. PNP drivers will typically go through the following sequence:

Þ	Issue the CM_GetVersion call to verify existance of a PNP configuration management utility

Þ	Issue the CM_GetConfig call to receive a valid configuration from the configuration management utility. Since CM_GetConfig  uses an index rather than device ID, the device driver will need to search for its specific instance. This mechanism actually allows a single driver to support multiple devices.

Þ	Parse the returned structure, determine assigned configuration and complete the initialization sequence

Refer to the example section for more information on driver initialization sequence.

# Data Structures

PNP clients will need to be familiar with the following data concepts:

·	Device ID - this ID must uniquely identify a specific device. The typical format of this ID is described below. The physical and logical device ID must be used when issuing the CM_LockConfig call.

[<bus ID>;<device ID>;<serial #>;<logical device ID>;<flags>]

where <bus ID> can be:

ISADEVICE		0x00000001

EISADEVICE	0x00000002

PCIDEVICE		0x00000004

PCMCIADEVICE	0x00000008

·	Bus access handle - the configuration utility handles only non-bus or device specific resources. The bus access handle allows drivers and installation programs to access information which is specific to their devices or their respective bus.

·	Configuration buffer - buffer containing the logical configuration which specifies the resources which have been assigned to the device.

## Device ID

Struct Device_ID {

DWORD	BusID			; bus type, 0 is undefined, input/output

DWORD	DevID			; physical device ID, 0xffffffff is undefined, input/output

DWORD	SerialNum		; instance or serial number, 0 is undefined, input is index

; output is serial number

DWORD	LogID			; Logical device ID for PNP ISA, Class code for PCI, 0xffffffff

; is undefined input/output

DWORD	Flags			; Bit 0: device has been initialized (1)

}

## Bus access handle

Union	Bus_Access {

struct	PnPAccess {

BYTE 	CSN			; Card slot number deternined during ISA isolation

BYTE	LogicalDevNumber	; Logical device number

WORD	Read_data_port		; Read data I/O port deternined during ISA isolation

}

struct	EISAAccess {

BYTE 	SlotNumber		; Card slot number

BYTE	FunctionNumber		; EISA sub-function number on multi function card

WORD  	reserved			;

}

struct	PCIAccess {

BYTE 	BusNumber		;

BYTE 	DevFuncNumber		;

WORD 	reserved			;

}

}

## Configuration buffer

The configuration buffer is used to describe configuration information for a certain device. The configuration buffer includes a generic configuration part (bus/device independent) and a vendor specific part which includes bus and device specific information.

#define	MAX_MEM_REGISTERS		9

#define	MAX_IO_PORTS			20

#define	MAX_IRQS				7

#define	MAX_DMA_CHANNELS		4

Struct Config_Buffer {

struct Device_ID	sdevice_ID_handle			; ID information

union	Bus_Access	ubus_access_handle			; info to allow config. access

WORD		wNumMemWindows			;  Num memory windows

DWORD		dMemBase [MAX_MEM_REGISTERS]  	; Memory window base

DWORD		dMemLength [MAX_MEM_REGISTERS]	; Memory window length

BYTE			bMemAttrib [MAX_MEM_REGISTERS] 	; Memory window Attrib

WORD		wNumIOPorts				;Num IO ports

WORD		wIOPort_Base [MAX_IO_PORTS]		; I/O port base

BYTE			bIOPort_Length [MAX_IO_PORTS]	; I/O port length

WORD		wNumIRQs				; Num IRQ info

BYTE			bIRQRegisters [MAX_IRQS]		; IRQ list

BYTE			bIRQAttrib [MAX_IRQS]			; IRQ Attrib list

WORD		wNumDMAs				; Num DMA channels

BYTE			bDMALst [MAX_DMA_CHANNELS] 	; DMA list

BYTE			bDMAAttrib [MAX_DMA_CHANNELS] 	; DMA Attrib list

}

## Resource attribute fields

### Memory Range

Plug and Play ISA:

Bit 0: 	0 - Use memory upper limit for decoding.

1 - Use memory range length for decoding.

Bit 2:1  	00 - 8-bit memory

01 - 16-bit memory

10 - 8 and 16-bit supported

11 - Reserved

EISA:

Bit 0:  	0 - Read only (ROM)

1 - Read/Write (RAM)

Bit 1: 	0 - Not cached

1 - Cached

Bit 2:   	0 - Cache write-through

1 - Cache write-back

Bit 4:3  	00 - SYS (base or extended)

01 - EXP (expanded)

10 - VIRtual

11 - Other

Bit 5    	0 - Not shared memory

1 - Shared memory

Bit 7:6  	Reserved

Bit 9:8  	00 - BYTE access

01 - WORD access

10 - DWORD access

11 - Reserved

Bit 11:10 	00 - 20 bit decode

01 - 24 bit decode

10 - 32 bit decode

11 - Reserved

Bits 15:12 	Reserved

### IRQ

Plug and Play ISA:

Bit 0:	1 - Level

0 - Edge

Bit 1: 	1 - Level high

0 - Level low

Bit 7:2 	Reserved

EISA:

Bit 4:0 	Reserved

Bit 5:  	0 - Edge Triggered

1 - Level Triggerred

Bit 6:  	0 - Not Shared

1 - Shared

Bit 7   	Reserved

### DMA

EISA:

Bit 5:0 	Reserved

Bit 6:  	0 - Not Shared

1 - Shared

Bit 9:7	Reserved

Bit 11:10 	00 - BYTE: 8-bit transfer, count by bytes

01 - WORD: 16-bit transfer, count by words

10 - DWORD: 32-bit transfer, count by bytes

11 - E-WORD: 16-bit transfer, count by bytes

Bit 13:12 	00 - ISA compatible timing

01 - Type A

10 - Type B

11 - Type C timing

# API

## Calling Conventions

The configuration management APIs can be called from DOS device drivers, TSRs, and applications, Windows 16-bit drivers, and Windows VxDs.  The APIs are identical for each of these, but the calling convention is different for VxDs.

The calling conventions for 16-bit code is the same for real mode DOS programs and protected mode Windows code.  The code should execute an Int 2Fh to detect the presence of the configuration management utility.  If the utility exists then it should make a far call to the entry point returned in ES:DI.

All registers are preserved except those explicitly modified as return values.

### 16-Bit Calling Convention

16-Bit code must obtain the entry point to the configuration utility by executing an Int 2Fh with the following parameters:

ES = DI = 0

BX = 34h (Configuration utility VxD Device ID)

AX = 1684h

If this Int 2Fh returns with ES:DI != 0 then the configuration utility exists and ES:DI contains the address to call.  The calling convention for all 16-bit code is to place the appropriate function code in AX and execute a far call to this entry point.  Note that the entry point returned from the Int 2Fh function will be different for code running in real and protected mode, but the services provided and the calling convention are identical (only VxDs have a different calling convention).

### VxD Calling Convention

VxDs can call the configuration VxD directly.  To test for the presence of the configuration management serivces a VxD should execute the CM_GetVersion call.  If the ID returned is EAX = 0 then the configuration management services are not present.  Otherwise, the configuration utility is present.  Below is a sample of how to check for the presence of configuration management services in a VxD:

VxDcall	CM_Get_Version

or	eax, eax

jz	ERROR_NO_CONFIG_MANAGER

## Device Driver Interface (DDI)

This chapter describes the full interface to the configuration managment utility.  The calling convention for 16-bit code and VxDs is described seprarately.

Presence Detection

#### 16-Bit Code

16-bit software must obtain the configuration utility entry point by executing the Int 2Fh described in the 16-Bit Calling Convention section of this document.  If the entry point is returned, the configuration utility is present.

#### Virtual Device Drivers

VxDs should call CM_GetVersion and check for a non-zero return in EAX.  Any non-zero return indicates the presence of the configuration utility.

CM_GetVersion

Returns the DDI version supported by the configuration utility.  VxDs also use this service to detect the presence of the configuration utility.

#### 16-Bit Code

Entry:

AX = 0000h

Exit:

AX = 0 indiates no configuration management utility installed

otherwise

AH = Major verison number (1)

AL = Minor version number (0)

BX = Number of deices identified by the configuration utility

#### Virtual Device Drivers

Entry:

None

Exit:

EAX = 0 indiates no configuration management utility installed

otherwise

AH = Major version number (1)

AL = Minor version number (0)

EBX = Number of deices identified by the configuration utility

CM_GetConfig

This function is used by drivers to obtain their configuration from the configuration management utility. The caller must allocate a buffer for the returned configuration. The configuration utility maintains an arbitrary logical ordering of the devices in a system and assigns each device a logical index starting with the value 0.  The typical method for accessing the configuration information for a specific device is to sequence through the device indices, starting at 0, until CM_GetConfig returns the configuration information for the desired device.  The device is identified by the device ID structure returned with the configuration information.

#### 16-Bit Code

Entry:

AX = 0001h

BX = Device index (starts with 0)

ES:DI -> Configuration buffer to receive config information

Exit:

If CX = 0 then success

ES:DI buffer contains config information

else CX = Error code

1 - Index out of range (used to detect end of device list)

#### Virtual Device Drivers

Entry:

EAX = Bus number

EBX = Device index (starts with 0)

EDI -> Configuration buffer to receive config information

Exit:

If ECX = 0 then success

EDI buffer contains config information

else ECX = Error code

1 - Index out of range (used to detect end of device list)

CM_LockConfig

This function is only called by setup programs. It is used by installation utilities to explicitly assign a configuration to a device.  This is typically used for assigning configurations to non-plug and play devices but could be used to statically configure a plug and play device.  The device ID and configuration details are contained in the input structure.  If the resources identified in the input structure are not available (i.e., the resources have already been assigned to another device), the status will signal failure of the call.  The assigned configuration will not take effect until the system is rebooted (i.e., this call does not cause the specified device to be configured).

#### 16-Bit Code

Entry:

AX = 0002h

ES:DI -> Configuration buffer

Exit:

If CX = 0 then success

ES:DI buffer contains assigned configuration

else CX = Error code

1 - Resources conflict

2 - Invalid configuration information

#### Virtual Device Drivers

Entry:

EDI -> Configuration buffer

Exit:

If ECX = 0 then success

EDI buffer contains assigned configuration

else ECX = Error code

1 - Resources conflict

2 - Invalid configuration information

CM_UnlockConfig

This function is used by installation utilities and device drivers to release the resources that had been previously explicitly assigned to a device with the CM_LockConfig call.  The released resources will immediately be available.  Thus, the installation utility must ensure that the device is no longer using the resources before releasing them.

#### 16-Bit Code

Entry:

AX = 0003h

ES:DI -> Configuration buffer

Exit:

If CX = 0 then success

ES:DI buffer contains assigned configuration

else CX = Error code

1 - Invalid configuration information

#### Virtual Device Drivers

Entry:

EDI -> Configuration buffer

Exit:

If ECX = 0 then success

EDI buffer contains assigned configuration

else ECX = Error code

1 - Invalid configuration information

# Example

This code is a sample of a typical initialization sequence for a DOS driver, TSR, or Windows 16-bit DLL or appliation.  The code will work both in real and protected mode.

.model	small

.stack	400h

;

;   Equates

;

CM_VxD_Device_ID    	EQU     034h

CM_GetVersion	EQU     0

CM_GetConfig	  	EQU     1

CM_LockConfig     	EQU     2

CM_UnlockConfig     	EQU     3

CM_ConfigBuffSize   	EQU     SIZE Config_Info_Struc

My_Bus_ID		EQU    1		; Just for example

My_Dev_ID	        	EQU     1234h   	; Just for example

My_Serial_Num	EQU     1		; Just for example

My_Log_ID		EQU     1234h	;

Max_Mem_Registers	EQU     9

Max_IO_Ports		EQU     20

Max_IRQs		EQU     7

Max_DMA_Chhannels	EQU     4

;

;   Configuration data structure

;

Config_Info_Struc STRUC

CIS_Bus_id			dd	?

CIS_Device_id			dd	?

CIS_Serial_Number		dd	?

CIS_Log_ID			dd	?

CIS_Flags			dd	?

;

;	NOTE:  For this example, we are using the ISA Plug & Play card access data only since the

;	sample driver is for a theoretical new ISA card.

;

CIS_CSN		db	?

CIS_LogicalDevNumver	db	?

CIS_Read_data_port	dw	?

;

;	Below this point is data common to all devices on all busses

;

CIS_NumMemWindows	dw	?

CIS_MemBase	dd	Max_Mem_Registers dup (?)

CIS_MemLenght	dd	Max_Mem_Registers dup (?)

CIS_MemAttrib	db	Max_Mem_Registers dup (?)

CIS_NumIOPorts	dw	?

CIS_IOPort_Base	dw	Max_IO_Ports dup (?)

CIS_IOPort_Length	db	Max_IO_Ports dup (?)

CIS_NumIRQs		dw	?

CIS_IRQRegisters	db	Max_IRQs dup (?)

CIS_IRQAttrib		db	Max_IRQs dup (?)

CIS_NumDMAs	dw	?

CIS_DMAList		db	Max_DMA_Channels dup (?)

CIS_DMAAttrib	db	Max_DMA_Channels dup (?)

Config_Info_Struc ENDS

;

;   Local data

;

.data

CM_Entry_Point	LABEL DWORD

CM_Entry_Point_Off  	dw	    ?

CM_Entry_Point_Seg  	dw	    ?

My_Config_Buffer    	Config_Info_Struc<>

My_Search_Buffer	LABEL DWORD

dd	My_Bus_ID

dd	MY_Dev_ID

dd	My_Serial_Num

dd	My_Log_ID

Search_Buff_Size	EQU	$-My_Search_Buffer	 ; Size of compare buffer

;

;   Sample Code

;

.code

Sample PROC NEAR

mov	ax, @DATA

mov	ds, ax			; Set up DS to data seg

xor	di, di			; Set to 0 to detect if

mov	es, di			; config utility not around

mov	bx, CM_VxD_Device_ID

mov	ax, 1684h

int	2Fh			; Ask for entry point

mov	ax, es

or	ax, di			; Q: Is ES:DI 0?

jz	ERROR_NO_CONF_UTILITY	;    Y: No config utility

;    N: Save it

mov	[CM_Entry_Point_Off], di

mov	[CM_Entry_Point_Seg], es

mov	ax, ds

mov	es, ax

xor	bx, bx			; Start with index 0

Sample_Search_Loop:

lea	di, My_Config_Buffer	; ES:DI -> Buff to get conf

mov	ax, CM_GetConfig

call	[CM_Entry_Point]		; Get my config

test	cx, cx			; Q: Success returned?

jnz	ERROR_DEV_NOT_FOUND	;    N: CX = Error code

mov	cx, Search_Buff_Size/2

lea	si, My_Search_Buffer

cld				; Make sure cmpsw works right!

repe cmpsw			; Q: Is this the right device?

je	Found_My_Device		;     Y: Stop looking.  Init device now

inc 	bx			;     N: Try the next one

jmp	Sample_Search_Loop

Found_My_Device:

;

; At this point My_Config_Buffer contains the configuration

; information for this device.	Initialize driver using

; returned configuration.

;

jmp	Sample_Exit

ERROR_DEV_NOT_FOUND:

ERROR_NO_CONF_UTILITY:

;

;   Do something to clean up for these errors

;

Sample_Exit:

mov	ax, 4C00h   		; Normally would be terminate and stay

int	21h	      		; resident.  This is normal DOS exit.

Sample ENDP

END	Sample

Only the APIs described in this specification will be supported in future versions of Windows and DOS