TPLODR.EXE

Syntax:

TPLODR <port#> <filename>

This command will load an updated ROM image from <filename> into the

T/Port <port#>. <port#> must be in the range 1-4 and the device at the

specifed port must be a Telcor T/Port type device. The integers 1 through

4 represent the hardware port addresses 03f8, 02f8, 03e8 and 02e8 in that

order. The filename is a concatenation of all of the .ROM (eg 1515pt1.ROM)

images for a given release. Each successive image occupies 32768 bytes

in the file. The loader program will search the specified file until it

finds an image that matches the T/Port to be loaded.

A successful cycle of this program will displays as follows:

Telcor T/Port downloader program

Tport ID string <xxxxxxxxxx>

File  ID string <yyyyyyyyyy>

File <filename> loaded into T/Port <port#>

Where <xxxxxxxxxx> and <yyyyyyyyyy> are the model and date strings

of the T/Port and the downlaoded file respectively. The format of

these strings is described in TPORT.DOC.

Program load failure reasons:

Invalid port number: <port#>

<port#> is not in the range 1 - 4.

Device on Port <port#> is not a T/Port

The device at <port#> is not present or

not a T/Port

T/Port <port#> does not have download capability

The Tport cannot be down loaded. It's

ROM must be replaced before download can

be performed.

Cannot open <filename> - T/Port not loaded

The specified filename cannot be found.

Invalid load file <filename>

The file <filename> is not acceptable

for a download for one or more reasons:

A) filesize is not modulo 32768, B) The

last 2 bytes of a ROM image are not page

self relative.

File <filename> is incompatible with T/Port

There is not a model image in the in

<filename> that is correct for the

specified T/Port.

Download failed - reset T/Port and try again

The T/Port failed to accept the download

within the allowed timeframe.