TPMAN DOCUMENTATION

===================

TPMAN is a program that will display the status of the RS-232 leads on the

specifed COM port.  It also allows the user to set the mode on Telcor's T/PORT

Serial Communications Coprocessor.  It's use is as follows:

-----

Install COM Port Monitor

------------------------

TPMAN <port #>

This command loads TPMAN as a memory resident program and displays the current

state of the RS-232 leads on the specified COM port.  The display is located in

the upper-right of the screen and can be toggled on and off by pressing the ALT

and LEFT SHIFT keys simultaneously.

Example:	TPMAN 3

The above command would load TPMAN and display the status of COM3.

-----

Uninstall COM Port Monitor

--------------------------

TPMAN U

This command uninstalls TPMAN from memory.

-----

Set Reduced Operation Mode (transmit buffer = 8 bytes)

------------------------------------------------------

TPMAN S <port #>

This command sets the T/PORT at the specified COM port to the reduced operation

mode.  Use this mode to make the T/PORT work with software that does not run

properly when the T/PORT is in the normal mode.

Example:  TPMAN s2	<- sets T/Port on COM2 to reduced operation mode.

-----

Set Normal Operation Mode (transmit buffer = 14,000 bytes)

----------------------------------------------------------

TPMAN F <port #>

This command sets the T/PORT at the specified COM port to the normal mode.  This

is the default mode and the way the T/PORT operates after it is reset or the PC

has been booted.

Example:  TPMAN f3	<- sets T/Port on COM2 to normal operation mode.

-----

Setting the RTS Flag

--------------------

TPMAN R <port #>

This command tells the T/Port to act as though RTS (request to send) is always

high.  This is useful when running software that does not raise the RTS signal.

Since the T/Port uses RTS/CTS flow control, it will not pass data to the PC

unless RTS is high.  Setting the RTS flag allows this type of software to work

with the T/Port.  If you are sending data out but not receiving data in, try

setting this flag.

Example:  TPMAN r1	<- sets RTS flag on the T/Port on COM1.