The T/PORT Advantage

Congratulations on purchasing the Telcor Systems T/PORT serial

communications coprocessor board. Your PC can now make the most of

your high speed communications equipment, as the T/PORT takes over

the time-critical chore of moving data in and out of the

communications port and delivers these important benefits:

Reliable Data Transfer

You can confidently transfer data using high-performance modems

and other asynchronous devices. You never worry about data loss,

as the T/PORT's intelligent microprocessor precisely controls

data flow to and from these devices.

Improved System Performance

You'll see significant throughput improvements as your CPU

can dedicate more of its resources to the running of complex

applications under Windows, UNIX and other multi-tasking

operating systems.

Greater Productivity

The T/PORT will help you improve your personal productivity. With

high-speed file transfers working completely in the background

where they belong, you can make more efficient use of your

system and spend less time monitoring data transfer operations.

What allows the T/PORT to outperform the standard communications chips

that were shipped with your PC?

First, the T/PORT has greater buffer capacity. Standard interfaces,

such as the 8250, 16450 and 16550 Universal Asynchronous

Receiver/Transmitters (UART), feature only 1 to 16 characters of

buffering, while the T/PORT dedicates 16,000 bytes to buffer

incoming data and 14,000 bytes to handle outgoing data.

Second, the T/PORT uses an onboard microprocessor and proprietary

firmware to flow control the modem (or other high speed serial

device connected to the T/PORT). In other words, if the

buffer becomes full, the T/PORT signals the modem to stop the

data flow (by lowering CTS, Clear to Send). When there is again

sufficient buffer space, T/PORT allows the modem to continue its

high-speed data transfer (by raising CTS). All of this is

accomplished without intervention from your PC's main CPU.

Installation

T/PORT Card installation is quite simple. It involves:

Selecting a COM port address and IRQ (Interrupt Request) line

Setting the DIP switches on the card for the selected port

address and IRQ line

Installing the card in a vacant expansion slot in your PC

Connecting and configuring the modem

Each of these steps is explained in the following pages. But if you

need any assistance in configuring the T/PORT, call us at

508-653-3999.

Selecting the COM Address

The first step is to examine your present system setup and decide:

Which COM port you will assign to the T/PORT

Since there can be more than one communications port in a PC, each

port must have its own unique address.

Which IRQ line you will assign to the T/PORT

There are at least four Interrupt Request (IRQ) lines used by

modems and other I/Odevices to alert the CPU to incoming data,

requests for data and other interactions. Devices can share an

IRQ line, as long as they don't use it at the same time.

The T/PORT Card can be configured to the following COM addresses

(shown with their standard IRQ lines):

Table 1: COM Port Address and Standard IRQ Lines

Serial Port Name    Address       Standard IRQ line

COM 1               3F8           IRQ4

COM 2               2F8           IRQ3

COM 3               2F8           IRQ4

COM 4               2E8           IRQ3

Note: You do not have to use the standard IRQ settings.

For example, you can assign IRQ5 to COM 3.

Note: If you are unsure about which COM ports are currently in use,

there are programs available that will examine your system for you,

including Microsoft's MSD program and the SYSINFO (also called SI)

program in Symantec's Norton Utilities.

To select a COM address:

Examine the configuration of all other COM ports in your PC

(both those that are built onto the PC's motherboard and those

that plug into expansion slots like the T/PORT). The program

called PORTEST.EXE on the Telcor support file diskette can be

very useful to identify the COM ports on your PC.

We suggest setting the T/PORT to the next available address. If

you are currently using all four addresses, you must remove or

disable one of the installed COM ports to make room for the

T/PORT.

Selecting an IRQ Line

Windows Users:

If you will be using the T/PORT under Windows exclusively and

not running DOS applications under Windows, the IRQ setting is

irrelevant because the T/PORT does not use interrupts when

running under Windows.

Set the T/PORT to the next available COM port and its standard

IRQ setting as listed in Table 1.

If you skip a port (e.g., use COM 4 when COM 3 is available),

insert this statement in your SYSTEM.INI file:

COM3IRQ=-1

which tells Windows that COM 3 is not in use, i.e. (there is NO

hardware with COM 3's standard address)

If you are also going to use the T/PORT under DOS (including a

DOS Window), the IRQ setting is important. Read the below.

DOS Users:

Set the T/PORT to the next available COM port and its standard

IRQ setting as listed in Table 1.

If you need to avoid an IRQ conflict, try using another

available COM port address or one of the non-standard IRQ lines

(IRQ5 or IRQ7).

If you select IRQ5 or IRQ7, make sure you configure your DOS

communications programs to use the specific IRQ line you have

selected.

Note: Remember that devices may share an IRQ line as long as they

are not both in use at the same time. For example, if you have

modems connected to both COM1 and COM3, both COM1 and COM3 may be

assigned IRQ4 as long as you do not try to use both modems

concurrently. If you plan to use two COM ports at the same time, you

may need to assign IRQ5 or IRQ7 to the T/PORT to avoid contention.

Configuring the T/PORT

The next step is to set the address and IRQ line using the four DIP

switches on the card. Follow this procedure:

Hold your T/PORT card horizontally, component side up, with the

metal mounting plate facing you.

The DIP switches are located next to the reset button (Figure 1). A

switch is in the UP position when it is switched toward the

component side of the board. It is in the DOWN position when it is

switched away from the component side.

Refer to Table 2 below and use switches 1 and 2 to set the COM

address. T/PortTwin users should see the installation manual for

their correct jumper settings.

Table 2 COM Switch Settings

COM Port   Switch 3        Switch 4

1          Down            Down

2          Up              Down

3          Down            Up

4          Up              Up

Refer to Table 3 and use switches 3 and 4 to set the IRQ line

Table 3 IRQ Switch Settings

IRQ        Switch 3        Switch 4

4          Down            Down

3          Up              Down

5          Down            Up

7          Up              Up

For example, to set the T/PORT for COM3/IRQ4 you would switch:

1 down, 2 up, 3 down, 4 down.

Now, re-boot your PC so it can learn the location of the T/Port.

The card must be reset any time the switch settings are changed.

Installing the T/PORT Card

To install the T/PORT card:

Turn the PC power OFF and unplug the power cord and, if neces-

sary, the data cables from the rear panel.

Remove the cover for access to the expansion slots. Locate an

empty expansion slot and remove its blank mounting plate from

the rear panel of the PC.

Insert the card in the empty slot and secure it to the rear

panel using the screw from the blank mounting plate.

Make sure the card is fully seated in its slot before placing

the cover on the PC.

Connect and Configure Your Modem

Connect your external modem or other serial Data Communi-

cations Equipment (DCE) with a standard (straight-through) cable

which has a female DB-9 (9-pin) connector at the T/PORT end.

The T/PORT Card expects the connected device to be configured to

support CTS/RTS flow control (also called CTS/RTS Handshaking

and Hardware Flow Control). The T/PORT card will assert and

control RTS and expects to see and obey CTS from the connected

device. The T/PORT card will pass on the state of DTR, RTS, DSR,

DCD and RI in the appropriate directions.

Note: CTS/RTS flow control is enabled on the T/PORT only when

DCD is HIGH.

When using the T/PORT card with modems, set the modem using the

configuration in Table 4. See your modem manual for details.

Table 4: Modem Configuration

Parameter                              Setting Common AT Command

Hardware Flow Control (RTS/CTS)         ON     AT&K3 (see note below)

Software Flow Control (Xon/Xoff)        OFF    AT&K3

DCD                                            AT&C1

Follows the true condition of the carrier (not forced true always)

DTR control                                    AT&D2

Disconnects (hangs up) when DTR goes from high to low

Note: Always verify the commands in your modem manual. If you will

be running communications programs under DOS only, you are ready to

begin. If you plan to run your communications programs under

Windows, read the section regarding Windows Software.

The RS-232 port of the T/PORT Card will communicate with your modem

at the speed, word size, parity and stop bits requested by your

communication program. Your modem's terminal port should be in an

auto-bauding mode or at the same speed that you have selected in

your communication program. These are standard requirements for

using a modem on a PC serial port. The T/PORT card will appear to

communications software as a 16450 UART chip (the type commonly

found in PC serial ports). However, the T/PORT card will far exceed

the performance of a 16450 or even a 16550 serial port interface.

Windows Software

If you will be using the T/PORT in a PC running Windows, you

need to replace the Windows 3.1 Communication Driver with the

Telcor T/PORT driver.

Insert the Telcor distribution diskette into an external drive

and copy the file TPORT.DRV to the Windows system directory

(usually, \WINDOWS\SYSTEM).

Using an editor, open the file SYSTEM.INI located in your main

WINDOWS directory (usually \WINDOWS).

Replace the line COMM.DRV=COMM.DRV with:

COMM.DRV=TPORT.DRV

to tell Windows to use TPORT.DRV as its communications driver.

Note: Because the TPORT.DRV driver disables interrupts when it

detects a T/PORT, the T/PORT does not consume an IRQ line when used

with Windows applications. You do not need to worry about IRQ

conflicts between the T/PORT and other devices in your PC

whilerunning Windows communications applications. For example, if

you have a serial mouse on COM2 using IRQ3, you can configure the

T/PORT for COM4 and IRQ3. Both the mouse and the T/PORT will work

fine under Windows. However, the T/PORT does use an IRQ line when

accessed from a DOS window.

Note: TPORT.DRV works with Windows 3.1 and up. It is not compatible

with Windows 3.0 or earlier versions.

Special Notes

115,200 bps users:

Due to some early confusion in the specifications from Microsoft

regarding the method for setting the COM port to a speed of

115,200 bps, there are several communications (PCPLUS for

WINDOWS is one) programs that use their own proprietary methods

for setting the COM port to 115,200 bps. Many of these

methods circumvent the TPORT.DRV communications driver

and therefore are not compatible with the T/PORT card.

If you are running without problems at speeds other than

115,200 bps but have trouble once you set your software

to 115,200 bps, call Telcor Technical Support

(508-653-3999) for more information.

Windows programmers:

The READ.ME file which accompaniesTPORT.DRV on the distribution

diskette explains how to program a T/PORT Card for 115,200 bps

operation.

TPMAN - T/PORT Utility Program

The program TPMAN.COM on the T/PORT disk is a DOS T/PORT utility

with five different command line parameters. This program is used

primarily as a diagnostic tool. Some situations may arise however

which require that certain TPMAN commands be placed in batch files

i.e. AUTOEXEC.BAT so that a T/Port port is in a certain non-standard

mode prior to running a particular communications program.

1.  TPMAN <com port #>                   e.g.: TPMAN 1

Loads TPMAN as a "memory-resident" program and displays the status

of the RS-232 leads of the specified port in the upper right

corner of the screen. When one of the six signal leads is High

(or True), a two-letter indicator will be displayed. When the

lead is Low (or False), its two-letter code will be missing from

the display. The codes are listed in Table 5.

2.  TPMAN U

Closes the program and removes TPMAN from memory.

3.  TPMAN S <com port #>                 e.g.: TPMAN S1

Sets the T/PORT at the specified COM port to the Reduced

Operation Mode in which the T/PORT uses a smaller (8 bytes)

transmit buffer. Use this mode with software that does not

run properly with the T/PORT in the Normal Operation Mode.

4.  TPMAN F <com port #>                 e.g.: TPMAN F1

Sets the T/PORT at the specfied T/PORT to Normal Operation Mode

in which T/PORT uses its full transmit buffer.

The Reduced Operation Mode of the T/PORT is necessary for some

programs to operate properly, especially those that require

responses back from the modem within a certain amount of time

Examples of programs that run better using this mode are

America Online, the Sierra Network and some fax software.

5. TPMAN R <com port #>                 e.g.: TPMAN R1

This command tells the T/Port to act as though RTS (request to send)

is always high.  This is useful when running software that does not

raise the RTS signal. Since the T/Port uses RTS/CTS flow control, it

will not pass data to the PC unless RTS is high.  Setting the RTS flag

allows this type of software to work with the T/Port.  If you are

sending data out but not receiving data in, try setting this flag.

Example:  TPMAN R1      <- sets RTS flag on the T/Port on COM1.

Table 5 TPMAN Lead Status Indicators

Symbol  RS-232 Lead             Source

Cd      Carrier Detect          Modem

Ri      RI Ring Indicator       Modem

Dr      DSR Data Set Ready      Modem

Cs      CTS Clear to Send       Modem

Rs      RTS Request to Send     T/PORT

Tr      DTR Data Terminal ready T/PORT

Troubleshooting

Most T/PORT installation problems stem from an incorrect

configuration. If your T/PORT card is not functioning properly

take these troubleshooting steps:

Review the sections regarding the selection of both the COM

address and the IRQ line.

Make sure that the T/PORT's COM address is unique. If there are

any other devices set for the same address as the T/PORT, it

will not work.

If you are unsure how the other devices in your PC are

configured, consult your PC dealer. Or, remove the cards that

you are not sure of and test the T/PORT again. If it works, you

can then replace the cards one-by-one until you determine which

card is conflicting with the T/PORT.

If you are running a DOS program, make sure that the T/PORT's

IRQ line is not being used by any other device in your PC. The

same procedures described above are useful in solving IRQ

conflicts as well.

Make sure that your communications software is set for the same

COM port address/IRQ combination that the T/PORT is set to.

If you are not using a standard COM port/IRQ combination (see

Table 1), you must specify in the software that you are using

the non-standard combination.

If you are running Windows, make sure that your COM port is

configured for the correct address and IRQ line. You can check

this using the Control Panel:

Select the Ports icon and then click on the COM port that

matches the T/PORT's setting.

Click on the Settings... button.

Click on the Advanced... button.

Confirm that both the Port Address and the IRQ setting are

correct. A value of default should be changed to the actual COM

port address and IRQ numbers.

If you are experiencing problems when running DOS communications

applications under Windows, try disabling the Windows Virtual

Communications Driver (VCD).

Edit your SYSTEM.INI file to change this line

DEVICE=*VCD  to

;DEVICE=*VCD

Then restart Windows.

The second most common installation problem involves the RS-232

interface between the T/PORT card and your modem.

Check to make sure the modem is using CTS/RTS hardware flow

control,or at least maintaining CTS true always.

If your modem isn't providing CTS, or the connecting cable has

no wire for the signal, you will not be able to communicate with

your modem because the T/PORT card will believe that it has been

flow controlled by the modem. If necessary, use an RS-232

breakout box or some other indicating device with LED displays

to show the high or low status of the various signals on the

cable between the serial port card and your external modem. Such

an indicator can be quite valuable when troubleshooting RS-232

problems.

The T/PORT card uses no flow control when the modem is in the

command state (as indicated by DCD low). The T/PORT card

switches to active CTS/RTS handshaking once it sees a true or

high DCD signal from the modem. It is important that your modem

not force DCD true always. DCD should only be true when you have

a good modem connection to another modem over a telephone line

(AT&C1 is the standard AT command for this configuration).

The signals necessary for proper operation of your modem and the

T/PORT card are listed in Technical Specifications.

Technical Specifications

��������������������������������������������������������������Ŀ

�                    DB-9 Pin Description                      �

��������������������������������������������������������������Ĵ

�  DB-9   � Signal                   �Source � DB-25           �

��������������������������������������������������������������Ĵ

�  Pin 1  � DCD  Detect Carrier      �DCE    � Pin 8           �

�  Pin 2  � RD   Receive Data        �DCE    � Pin 3           �

�  Pin 3  � TD   Transmit Data       �T/PORT � Pin 2           �

�  Pin 4  � DTR  Data Terminal Ready �T/PORT � Pin 20          �

�  Pin 5  � SG   Signal Ground       �Common � Pin 7           �

�  Pin 6  � DSR  Data Set Ready      �DCE    � Pin 6           �

�  Pin 7  � RTS  Request to Send     �T/PORT � Pin 4           �

�  Pin 8  � CTS  Clear to Send       �DCE    � Pin 5           �

�  Pin 9  � RI   Ring Indicate       �DCE    � Pin 22          �

����������������������������������������������������������������

Buffers & Ports

Transmit buffer    14K bytes

8 bytes in reduced operation mode

Receive buffer     16K bytes

Port               1 standard DB-9 RS-232 port

Power              Standard 8 bit IBM style PC I/O slot power

(+5 VDC, +12 VDC, -12 VDC)

Warranty Information

Telcor Systems Corporation (TELCOR) warrants that this product will

be free from defects in material and workmanship under normal use

and service for a period of three years.

If, during the Warranty Period, any component part of this product

becomes defective by reason of material or workmanship, TELCOR will

repair or, at its option, replace the product provided that (1) the

product has not been subjected to misuse, neglect, accident or

improper installation, (2) the product has not been repaired,

altered or modified by any persons other the TELCOR's own factory

service personnel and (3) the product has been returned to TELCOR's

factory and all transportation charges for such return have been

prepaid.

TELCOR makes no warranty of any kind, expressed or implied, except

as herein above stated. By accepting the product, except as

otherwise provided herein, the buyer assumes all risk and

liability resulting from the use of the product whether used

separately or in combination with other equipment, and agrees that

the provisions of the warranty contain the buyer's sole right and

remedy. TELCOR neither assumes nor authorizes any person to assume

for TELCOR any other liability in connection with the sale or use

of the product.

Note

����������������������������������������������������������������

This Warranty excludes damage incurred in shipment. Should

equipment be received in damaged condition, contact the carrier

immediately. All claims for damage in transit should be filed

with the carrier.

����������������������������������������������������������������

FCC Information: Radio & Television Interference

This equipment has been tested and found to comply with the limits

for a Class B digital device, pursuant to Part 15 of the FCC rules.

These limits are designed to provide reasonable protection against

harmful interference in a residential installation. This equipment

generates, uses, and can radiate radio frequency energy and, if not

installed and used in accordance with the instructions, may cause

harmful interference to radio communications. However, there is no

guarantee that interference will not occur in a particular

installation. If this equipment does cause harmful interference to

radio or television reception, which can be determined by turning

the equipment off and on, the user is encouraged to try to correct

the interference by one or more of the following measures:

Reorient or relocate the receiving antenna.

Increase the separation between the equipment and the receiver.

Connect the equipment into an outlet on a circuit different from

that to which the receiver is connected.

Consult the dealer or an experienced radio/TV technician for

help.

You may also find helpful the following booklet, prepared by the

FCC: How to Identify and Resolve Radio-TV Interference Problems,

which is available from the U.S. Government Printing Office,

Washington, DC 20402

Changes and modification not expressly approved by the manufacturer

or registrant of this equipment can void your authority to operate

this equipment under Federal Communications rules.